Abstract

Introduction

Road safety analysis relies heavily on statistical models that quantify relationships between contextual factors, traffic settings, roadway characteristics, and crash frequency or severity (Butt, 2025). Last years there has been an increasing interest in analysis methods that involve statistically rigorous models that facilitate reproducibility, associations, and causal interpretations. In addition, there is a necessity in the application and development of methods that provide the ability to capture the marginal safety effect of individual geometric design elements while controlling traffic exposure, unobserved heterogeneity, and spatial or temporal dependence (Vayalamkuzhi, 2016).

In this sense, contemporary road-safety research has predominantly relied on Safety Performance Functions (SPFs), typically specified within count-data modelling frameworks such as Poisson, Negative Binomial, and hierarchical extensions, to estimate expected crash frequencies under varying geometric and operational conditions (AASHTO, 2010; Venkataraman, 2014). However, the suitability and inferential validity of these methods vary substantially across applications, particularly when assessing isolated Design Exceptions (DEs) on existing roadway networks, where limited variation, selection bias, and confounding pose significant econometric challenges.

Among the frameworks and methods applied, the ones proposed by the Highway Safety Manual (HSM) have become a standard approach for developing SPFs and Crash Modification Factors (CMFs) (AASHTO, 2010). In this formulation, crash frequency is typically expressed as a multiplicative function of a baseline SPF and a set of CMFs that adjust for geometric design, operational features, and control characteristics (Chen, 2014).

Despite its widespread use, the traditional HSM formulation presents several limitations. First, exposure, commonly measured through traffic volume such as Annual Average Daily Traffic (AADT), is typically treated externally or through fixed offsets, implicitly assuming a constant elasticity between traffic and crash frequency (Gayah, 2021). However, both empirical and theoretical studies suggest that this relationship tends to be non-linear and context-dependent, often exhibiting differences when traffic increases (Gayah, 2021). As a result, models with fixed exposure elasticity as assumption may fail to capture important variations in crash risk across different roadway and environmental conditions.

Second, CMFs in traditional frameworks are generally estimated through multi-stage procedures, where a baseline SPF is first calibrated and then adjusted using multiplicative factors. This separation may introduce inconsistencies and omitted variable bias, particularly when exposure and contextual factors are not adequately accounted for in both stages. In practice, this can lead to biased estimates of treatment effects where geometric design may absorb the influence of traffic of environmental conditions (Wu, 2017).

Third, traditional regression-based approaches often rely on complete pooling or fixed-parameter specifications, assuming that the data-generating process is homogeneous across all observations. However, crash data are inherently structured, with observations nested within road segments, functional classes, or environmental contexts. Ignoring the hierarchical structure can lead to biased inference and unstable estimates, especially in the presence of unobserved heterogeneity (Wali, 2018).

To address these limitations, this study analyzes the use of hierarchical models for crash frequency and road safety analysis. This setting integrates exposure, geometric design, and contextual factors within a unified specification. Furthermore, this approach extends traditional models by allowing exposure to enter the model as a varying coefficient, enabling the elasticity of crashes with respect to traffic to depend on contextual factors (Chen, 2014). This formulation explicitly separates baseline crash propensity from traffic-response behavior, providing a more flexible and behaviorally consistent representation of crash generation mechanisms.

Along with this advantage, the framework has the ability to derive CMFs directly from a one-stage model. Unlike traditional approaches that treat CMFs as fixed multiplicative adjustments, the hierarchical model embeds treatment effects within the same structure as exposure and contextual variables. As a result, CMFs can be interpreted as functions rather than constant, allowing their effects to vary across traffic levels and environmental conditions. This provides a more realistic representation of safety impacts, particularly for interventions whose effectiveness depends on exposure.

The framework is evaluated using three empirical datasets: the Washington state dataset, the Queensland Heavy Vehicles dataset, and a rural roads dataset from Maine. These case studies allow for the assessment of the model under different levels of heterogeneity and data structures. The results show that the hierarchical approach improves predictive performance, stabilizes estimates, and provides more interpretable and behaviorally consistent CMFs compared to traditional specifications.

Overall, this study contributes to the road safety literature by analyzing a unified modeling framework that bridges SPF and CMF development, incorporating exposure in a flexible manner, and accounts for structured heterogeneity in crash data. The findings have important implications for both methodological development and practical applications in road safety management and infrastructure design.

Literature review

Multilevel modelling in traffic safety analysis

The development of fully specific models that capture all variables and interactions between the covariates in the design matrix (features) is still a challenge for road safety analysis and geometric design of highway roads. The traditional crash prediction model proposed by the Highway Safety Manual (HSM) has the structure shown in Equation (1) (Bahar, 2014; Elvik R. , International transferability of accident modification functions for horizontal curves, 2013):

Np=Nspf,x×Cx×i=1kCMFi,x      (1)

Where Np is the predicted average crash frequency for a specific year for a site of type x, the term Nspf,x is the predicted average crash frequency determined for base conditions in the site of type x (e.g., a function of characteristics of the road), CMFi,x corresponds to the Crash Modification Factor (CMF) or Crash Modification Function (CMFunction) for specific factor i for site type x, the term Cx corresponds to a calibration factor that adjust the Safety Performance Function (SPF) for local conditions for site type x.

In (1), a CMF or CMFunction is a multiplicative factor that reflects the expected change in safety performance associated with the corresponding change in some geometric design, control feature, or operational characteristics of the roads. In (1) because CMFs are multiplicative, exposure in the model is treated externally, and the elasticity of this exposure is fixed, as such equation (1) does not explicitly separate baseline crash propensity from traffic-response behavior. Where:

Crash Risk=Baseline Propensity×Traffic Scaling

It is worth noting that the development of CMFs for road safety analysis involves a set of methodological steps that validate their statistical properties (Elvik R. H., The handbook of road safety measures, 2009).

Within the framework of development of CMFs for road safety, regression-based approaches appear as one of the methodologies for modeling such features and their interactions. Among the methods related to regression-based, multilevel analysis is one of the approaches that present a set of characteristics that make them interesting for road safety.

A multilevel model is a regression model in which the parameters to estimate has a probability distribution, in contrast to traditional methods in which the estimates are fixed. Examples of the use of such models for road safety can be found in terms of CMF development, severity of injuries in road crashes, and to model crash outcome probabilities for vehicles and pedestrians (Huang H. &.-A., 2010; Chen, 2014; Huang H. C., 2008; Jones, 2003; Kim, 2007; Park, 2019).

Let i=1,…,n index theset of observations in the sample. Let the outcome variable (crash frequency) yi∼Expontential Family(⋅). The conditional mean is defined as gμi=xiTβ μi= E[yi|xi], and is realted to the covariates through a line function g⋅ such that such that gμi=xiTβ, (where xi∈Rp a set of covariates for observation i, and β a set of coefficients to be estimated for the model (Gelman, 2007).

As stated above, a hierarchical model assumes the β coefficients as non-fixed for explaining the outcome. Let j=1,…,J be the group to which observation i pertains. Furthermore, the link function can be written as gμij=xijTβj. Note that the coefficients have the suffix j, denoting they are not fixed and can vary across groups j∈J. For instance, if we let the intercept to vary, the model can be written as gμij=β0j+xijTβfixed. Where β0j∼N(ζ0,τ2), where the terms ζ0,τ2 are estimated directly from the data.

On the other hand, if we let to vary the slopes, we can write the model as β0j=γ00+μ0j and β1j=γ10+μ1j, such that:

μ0jμ1j∼N0, τ02    ρτ0τ1ρτ0τ1  τ12    (2)

As a result, for a two levels model, we have that:

gμij=xijTβ+zijTμj     (3)

Where the left term represents the fixed part, and the right term represents the random part of the model. Furthermore, μj∼(0, Σ) are the random effects for group j, and zij are the subset of covariates where the coefficients are random.

An analysis conducted by (Huang H. &.-A., 2010) considers a two-level model as a random parameters model in the following form shown in Equations (4)-(5). In terms of road safety, this hierarchical approach tackles the clustered structure of crashes. Traditional models assume that units are independent. However, traffic safety data often violates this assumption because observations are generated within structure contexts, such as site type, functional class, or another grouping variable (Pervaz, 2022)

Level 1:f-1θ=β0L1+xL1βL1+εL1    (4)

Level 2: β0L1=β00L2+xL2β0L2+ε0L2     (5)

Such that:

βL1=β01L2+xL2β1L2+ε1L2     (6)

From level 1 in Equation (4) and the sublevel in Equation (6) we have that:

f-1θ=β0L1+xL1βL1+εL1=β0L1+xL1β01L2+xL2β1L2+ε1L2+εL1  (7)

Later, taking βL1, we have that:

f-1θ=β00L2+xL1β01L2+xL2β0L2+xL1xL2β1L2+εL1+ε0L2+xL1ε1L2   (8)

In this structure, each component denotes a characteristic from the structure of the entire model, specifically:

- β00L2: Corresponds to the intercept when the rest of variables are zero. This is the mean value of all covariates.

- xL1β01L2: Effect of the covariates on level 1.

- xL2β01L2: Effect of the covariates on level 2.

- xL1xL2β1L2: Effect of the interaction between levels. This term allows the effect of lower-level terms to depend on upper-level terms in a cross-level interaction form.

Furthermore, regarding the random effects, we have:

- εL1: Error terms are associated with the level 1 structure.

- ε0L2: Error terms are associated with the level 2 structure. Individuals from level 1 that pertain to the same group in level 2 share the variance. This results in a within-group covariance. All of it allows to write the intercept to vary across groups and observations, and takes the following form:

Intercept=β00L2+εL1+ε0L2    (9)

- xL1εL2: Vector of the intercept of the variables in level 1 associated with level 2. This makes the slope of xL1 to vary across groups. Now, the slope can be written as Equation (10):

Slope=β01L2+ε1L2    (10)

- This structure allows us to write the model depending on varying intercept, varying slopes, or both, which depends on the scope of the analysis and data heterogeneity that allows for integrating random parameters.

Properties of random parameters models

As noted by (Huang H. &.-A., 2010), the hierarchical model has a set of properties that are beneficial in road safety analysis, specifically:

- Simultaneous modeling that incorporates data from individuals and groups of individuals (clustered data, observations that are not fully independent, or groups of interest such as roads, states, types of highways).

- Efficient parameter inference. Compared to complete pooling and no-pooling (the two traditional approaches for regression-based analysis), the hierarchical model yields a partial-pooling which is a mixture of the two first ways of modeling groups of data integrating group-specific structures along with considerations of the entire set of information.

- Hierarchical models improve estimation efficiency by sharing information across groups, making it possible to obtain robust estimates for small samples while preserving inference for the overall population.

Regarding property ii., related to the efficiency of parameter inference, let J be a set of groups, such that j∈J, and nj are the observations for group j. Furthermore, yij|λj∼Poissonλj, i=1,…,nj, where the term λj denotes the rate of occurrence of events (crashes) per group. We assume that if follows a Gamma distribution (λj∼Gammaα,β), which yields a Poisson-Gamma mixture model with α,β be the shape and the rate of the Gamma distribution, which are estimated from the observed data. All of it results in the expression shown in (11) and (12) for the expected value and variance of the model.

Eλj=αβ    (11)

Varλj=αβ2    (12)

As stated above, there are three ways for modeling data in road safety. The first one consists of a no-polling approach, in which we ignore the grouping structure and the estimation of λj is conducted using the information for each group j independently (Gelman, 2007). Formally we say:

λjno-pooling=yi=1nji=1njyij   (13)

Which yields:

Eλjno-poolingλj=λj    (14)

Varλjno-poolingλj=λnj     (15)

Note that the variance shown in expression (15) can be high if the number of observations in the group j is small.

In contrast, in the case of complete pooling is applied, which ignores the grouping structure and the model is fitted for all the observations, the estimator is shown in (16).

λcomplete=1j=1Jnjj=Ji=1njyij=j=1JnjyjNt    (16)

Where Nt denotes the overall sample size. This estimation yields a set of properties that are of interest depending on the data structure, specifically:

Eλcompleteλ=λ    (17)

Varλcompleteλ=λNt     (18)

As a result, the variance decreases as the sample size increases, but it does not depend on the size of each specific group nj, in contrast to no-pooling. As a result, an issue derived from this approach is that if the data generation process has different parameters for each group, this is, specific-group rates λj, then the complete pooling introduces omitted variable bias. Let the true group means be λj, with overall average on the form λ=1Jjλj, then:

Eλcompleteλ=λ     (19)

Which is achieved only if the groups are balanced, this is λj≈λk, ∀j≠k. Furthermore, for any specific group j, the estimator is biased:

Biasλcomplete=λ-λj     (20)

And this bias can be high when the groups are highly heterogeneous, this is λi≁λj,∀i≠j. All of it yields that complete pooling is the most restrictive method as it assumes all variation between group is zero and considers the same data generation process for all units in the sample. In terms of road safety this would mean that each road segment, intersection, city, or urban context has the exact same crash rate after accounting for observed covariates, which is mostly unrealistic in real settings (e.g., rural vs urban roads, single carriageway vs double carriageway, different types of weather for different road segments).

Key differences between pooling types for regression-based models

As we stated above, the model with partial pooling can be set given that the Gamma distribution is conjugated, so the distribution of λj, given the observed data is shown in Equation (21) (German, 2025):

λj|yj∼Gammaα+i=1njyij, β+nj     (21)

So, the posterior mean, assuming α and β estimated from the data, can be set as shown in Equation (22) (German, 2025):

λjpartial=α+iyijβ+nj=α+njyjβ+nj   (22)

Let μ0=α/β be the prior-mean, we can write the partial pooling estimator as follows in (23):

λjpartial=njβ+njyj+ββ+njμ0    (23)

Now if we define a shrinkage operator on the form shown in (24):

wj=njβ+nj    (24)

We can write the weighted estimator on the following form:

λjpartial=wjyj+1-wjμ0    (25)

In Equation (25), as nj decreases, the shrinkage towards the prior-mean μ0 increases. Furthermore, note that wj∈(0,1), which implies the following property regarding the variance under the  partial pooling structure:

Varλjpartial=wj2λjnj<λjnj=Var(λjno-pooling)      (26)

The magnitude of the variance reduction  is directly determined wj2, which depends on the group-sepecific sample nj and the parameter β estimated from the data.

Comparing the variances under the different pooling structures (Equations (18) and (26)), yields the following result:

ΔVar=λ1Nt-nj(β+nj)2    (27)

Complete pooling combines all observations across groups to estimate a single paramater. Consequently,  its variance is primary driven by the total sample size, which is typically large. In contrast, partial pooling retains some group-level variability, resulting in a larger variance for a given group (unless one specific group dominates the sample, which can be rare).

Another important distinction arises in terms of the  Mean Squared Error (MSE). Recall that the MSE can be defined as a composition between variance and bias (Hodson, 2021):

MSE=Var+Bias2    (28)

Thus, the MSE of the no-pooling estimator can be written as:

MSEno-pooling=λjnj+0=λjnj      (29)

And the variance of the partial-pooling estimator is:

MSEpartial=wj2λjnj+(1-wj)2(μ0-λj)2        (30)

Therefore, the difference in MSE between the no-pooling and partial pooling can be defined as:

ΔMSE=MSEno-pooling-MSEpartial       (31)

ΔMSE=λjnj-wj2λjnj-(1-wj)2μ0-λj)2      (32)

ΔMSE=1-wj2λjnj-(1-wj)2(μ0-λj)2     (33)

As a result, the hierarchical model tends to have less MSE, especially for groups with fewer observations (derived from the property that it uses partial and complete information to make inferences on the estimators), yielding better inference and reliable results for groups with fewer observations compared to the no-pooling approach. Now, in terms of the complete pooling we have the following difference related to the MSEs:

MSEcomp-MSEjpartial≈λjNt-w2λjnj +((λ-λj)2-1-wj)2μ0-λj2>0 (34)

Hence the hierarchical approach yields strictly lower MSE compared to complete pooling, especially for sites with few observations.

Connection with Empirical Bayes

A property derived from the structure of hierarchical modeling is its connection with Empirical Bayes (EB), a method widely used in road safety analysis to estimate the outcome of design changes on roads and highways. The EB method is conducted (traditionally) on the following way: (i) Estimation of the distribution of parameters through (Maximum Likelihood Estimation) MLE (or other procedures such as partial MLE, OLS, or simulation), (ii) Given the estimations, the posterior distribution is adjusted considering observed results (German, 2025).

Recall the model described above has yij|λj∼Poisson(λj), while λj∼Gamma(α,β), with μ0=α/β, and Variance=α/β2. In an EB approach, the parameters α,β are estimated from the marginal distribution of the data. Later, for each group j, the posterior mean is calculated given α,β, taking the form shown in Equation (35).

λjEB=α+iyijβ+nj    (35)

Which is the same estimator from the partial-pooling procedure shown in Equation (25), but with α,β estimated differently in the EB method.

Development of SPF and CMF

A key property of the hierarchical modeling approach is its connection for developing SPF and CMF following the recommendations made by HSM. Recall the structure of the hierarchical model shown in Equations (7) and (8).

f-1θ=β00L2+xL1β01L2+xL2β0L2+xL1xL2β1L2+εL1+ε0L2+xL1ε1L2   (36)

This structure in (36) can be written as a varying-coefficient model in the multiplicative form shown in Equation (37):

Np=A×xB

Where A=α0×exp⁡(α1z1) and B=β0×exp⁡(β1z2) where x,z1,z2 are covariates for a varying-coefficient model. For the purposes of this analysis, we set x to be exposure terms (Annual Daily Average Traffic (AADT)), z1 characteristics of interest, and z2 are local factors of road segments. Given that, we can write the model (37) in a more detailed way as follows:

Np=α0×expα1z1xβ0expβ1z2   (38)

Taking logarithms on both sides it yields:

lnNp=α0+α1z1+β0expβ1z2lnx    (39)

Where the term β0expβ1z2lnx denotes the cross-level interactions presented in the data. Recall the HSM suggests the development of SPF in the following multiplicative form:

Np=NSPF×CMFi×…×CMFk×C=α0×expα1z1xβ0expβ1z2   (40)

In this setting, the base SPF has the form of the expression xβ0exp(β1z2), allowing to capture interactions between the x term (exposure) and local factors. Now, the term α0×exp⁡(α1z1) replaces the product of CMFs and the calibration factor (α0). This setting comprises all the structure proposed by the HSM in the development of SPF and CMFs, avoiding the use of external calibration (e.g., two stage methods) in the analysis.

Furthermore, to retrieve the CMFs from the expression, we have α0 as the multiplicative base factor when the rest of covariates are zero. But for this case, the SPF includes the exposure terms x, so α0 cannot be interpreted directly.

In this sense, the term exp⁡(α1) is the multiplicative factor of varying z1. Suppose that we go from z10=a to z11=b, according to the procedure suggested by the HSM, we can write the following:

Na=α0expα1axβ0expβ1z2     (41)

Nb=α0expα1bxβ0expβ1z2    (42)

Which is a change in the z1 term keeping all the rest of features constant. Now, the HSM retrieves the CMF as the change of a design feature or covariate measured as the ratio in the following form (AASHTO, 2010):

CMFa→b=NbNa=α0exp⁡(α1b)xβ0exp(β1z2)α0exp⁡(α1a)xβ0exp(β1z2)=exp⁡(α1b)exp⁡(α1a)=expα1b-a   (43)

Omitted variable bias

Note that the formulation of the SPF along with the CMF in one-stage approach shown in (38) includes the exposure term x. If we adjust a model ignoring this term, the model will depend entirely on design factors, generating omitted variable bias and making the effect of crashes on geometric features ignoring the effect of contextual factors.

In this sense, the literature has stated that exposure and contextual factors are determinants on the frequency of crashes. Omission of such terms means that the coefficients derived from local factors absorb all the effects from traffic, generating over-estimation or under-estimation of such effects (AASHTO, 2010; Lord, 2010).

For instance, if increasing exposure produces more crashes, along with increasing the probability of having more turning lanes, the model will attribute that increasing the number of turning lanes increases the number of crashes, when the number of lanes reduces crash frequency. This is an example of confounding. This setting yields that derived CMFs will be implicitly correlated with exposure and contextual factors not considered in the modelling setting, biasing the causal or associated effect of the treatment. This later effect is also called the Simpson paradox, in which a hidden third factor influences both variables (or the grouping and the outcome), creating misleading aggregate results (Elvik R. H., Basic concepts of road safety research, 2009).

Relation with the offset

In the multiplicative structure of the SPF-CMF model, the exposure x works in a different way compared to traditional offsets in count models. A model with offset takes the following form:

logμ=logAADT+xTβ    (44)

This structure leaves the offset term with an elasticity of 1 (its parameter β is fixed to this value). In the multiplicative hierarchical structure shown, the exposure is applied with variable elasticity (depending on contextual factors).

offset=β0expβ1z2lnx   (45)

Under the Hierarchical structure:

log⁡(μ)=(baseline)+(γ0+γ1z)log⁡(AADT)
In this formulation, γ0 represents the baseline traffic elasticity, while γ1allows the elasticity of crashes with respect to AADT to vary as a function of the contextual variable z. Consequently, traffic scaling is not fixed but depends on roadway or environmental characteristics, enabling exposure effects to differ across contexts.

This aligns with the literature stating that the frequency of accidents tends to increase less than the proportion of exposure increases due to saturation effects (Elvik R. , The non-linearity of risk and the promotion of environmentally sustainable transport, 2009; Fiorentini, 2025; Wagner, 2021).

Relationship between multilevel hierarchical model and multiplicative model with varying-coefficients

The general hierarchical model has two main parts, level 1 and level 2 structures. Level 1 (observation level) defines the linear predictor with the expression f-1(θ) as a function of level 1 predictors xL1 and coefficients to estimate on the form β0L1, βL1, taking the following form:

f-1θ=β0L1+xL1βL1+εL1    (46)

The level 2 (group level) specifies that the level 1 coefficients themselves are linear functions of level 2 predictors xL2 along with random terms, taking the following structure:

β0L1=β00L2+xL2β0L2+ε0L2     (47)

βL1=β01L2+xL2β1L2+ε1L2     (48)

The substitution of the terms in both levels yields a combined linear predictor that includes cross-level interactions xL1xL2β1L2 and a composite random term εL1+ε0L2+xL1ε1L2. The crash modification hierarchical model takes the form:

N=A(AADTmaj)B(AADTmin)C    (49)

Taking logs at both sides (since crash counts are modelled with a log-link Poisson) gives:

logN=logA+B(logAADTmaj+ClogAADTmin     (50)

Thus, in the notation of the general model, we can associate:

- f-1θ=log⁡(N)

- XL1 with log⁡(AADTmaj) and log⁡(AADTmin)

- β0L1 as the intercept (log⁡(A))

- βL1=(B,C) as the slopes for the two exposure variables

The sub-level CMFunctions can be written as logA=logα0+α1z1, logB=logβ0+β1z2, and logC=logγ0+γ1z2. Substituting the terms of the sub-level into the level 1 equation gives the following expression:

logN=logα0+α1z1+logβ0+β1z2logAADTmaj+logγ0+γ1z2logAADTmin    (51)

If we expand this expression it yields:

logN=logα0+α1z1+logβ0logAADTmaj+logγ0logAADTmin+β1z2logAADTmaj+γ1z2logAADTmin    (52)

Note that in the expression the fixed part contains the main effects of z1 (through the expression A and the cross-level interactions between z2 and the exposure terms. It is worth noting that the expression is log-linear for the level 2 equations for intercept and slopes.

Experimental setting

The experimental setting consists of the comparison between specification found in literature and models with partial pooling (hierarchical), and complete pooling. The complete pooling models are specified as considered variables with statistical significance during the fitting procedure, so non-significant variables are removed from the result. Three datasets containing crash frequency, geometric features, and contextual variables are used in this step.

The comparison of the results for the Washington dataset is shown in Table 1. The specification which is used as benchmark is found in (Simon Washington, 2020), and considers as features the width per lane in the segment (in feet), the number of curves, the tangent length, the number of interchanges, the maximum grade difference (in %), the shoulder width (in feet), the friction (0 to 100 with 100 being high), the expose (the annual average daily travel multiplied by the length of the segment), an indicator function if the road segment is considered as low-precipitation, an indicator function if the road segment is considered as high-snow, and an indicator function if the slope of the segment is flat.

The results shown in Table 1 reveal clear improvements in both statistical performance and interpretability of effects. While the intercepts are not directly comparable due to reparameterization and the presence of multiple levels in the hierarchical models, the latter introduces a richer structure with additional geometric variables, which helps mitigate omitted variable bias.

Several effects are consistent across models, such as the positive impact of the number of interchanges on crashes, although the hierarchical estimates a stronger magnitude, suggesting better identification of risk. A notable difference arises in the number of curves, where the literature reports a negative effect, but the hierarchical model finds a positive one. This reversal is likely due to the inclusion of additional controls, making the hierarchical estimate more behaviorally plausible, as more curves generally increase crash risk.

Other coefficients in the hierarchical model align well with expectations, such as negative effects for lane and shoulder widths (which indicate safer conditions) and a negative effect for flat slopes.

Regarding goodness of fit, the hierarchical model outperforms the literature model with a better predictive performance, with large reductions in MSE (313 vs 792), MAE (17.69 vs 28.13), and RMSE (9.85 vs 16.88), demonstrating that the hierarchical approach is more accurate in prediction.

Finally, whereas the literature model relies on random parameters to capture heterogeneity, the hierarchical model captures structured variability through multi-level deviations, offering a more principled representation of unobserved heterogeneity and reinforcing its advantages for CMF development.

Table 1: Specification found in the literature compared to the hierarchical framework for the Washington dataset.

| Specification provided by (Simon Washington, 2020) |  |  | Specification found through the hierarchical approach |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Estimate | p-value | Parameter | Estimate | p-value |
| α0 | 2.81 | <0.0001*** | α0 | -10.3774 | <0.0001*** |
| 1 if the segment is considered as low-precipitation, 0 otherwise | -0.41 | <0.0001*** | Width per lane | -0.0534 | 0.019** |
| Grade breaks in the segment | -0.05 | 0.02** | Number of curves | 0.0954 | 0.0009*** |
| Friction | -0.01 | 0.05* | Tangent length | 0.1014 | 0.0606* |
| Exposure | 2.70 | <0.0001*** | Number of interchanges | 0.4192 | <0.0001*** |
| Number of interchanges | 0.15 | 0.01** | Maximum grade difference | 0.0769 | 0.0855* |
| Number of curves | -0.16 | <0.0001*** | Shoulder width | -0.4338 | 0.0411** |
| 1 if the segment is considered as high snow | -0.18 | 0.01** | β0 | 5.3944 | <0.0001*** |
|  |  |  | 1 if the slope of the segment is flat, 0 otherwise | -0.0769 | 0.0635* |
| Random parameters |  |  |  |  |  |
| Exposure (Std. Dev.) | 0.95 | <0.0001*** | Std. Deviation (first level) | 0.5558 | <0.0001*** |
| Number of interchanges (Std. Dev.) | 0.53 | <0.0001*** | Std. Deviation (sub level) | 0.2381 | <0.0001*** |
| Number of curves (Std. Dev.) | 0.10 | <0.0001*** |  |  |  |
| 1 if the segment is considered as high snow (Std. Dev.) | 0.60 | <0.0001*** |  |  |  |
| Goodness of fit |  |  |  |  |  |
| BIC | 1993 |  | 2034 |  |  |
| Log-Likelihood | -956 |  | -986 |  |  |
| MSE | 792.52 |  | 313.01 |  |  |
| MAE | 28.13 |  | 17.69 |  |  |
| RMSE | 16.88 |  | 9.85 |  |  |

Figure 1 shows the fitted values from both the specification found in literature and the hierarchical framework. Both models capture a positive relationship between AADT and crashes, which is consistent with exposure theory on road safety analysis. However, the literature model shows much higher dispersion, especially at medium and high levels of AADT, along with many extreme fitted values and larger vertical spread for similar AADT values. As a result, this model is more sensitive to noise or omitted variables in the specification, likely to overreact to combinations of covariates, which suggests unstructured heterogeneity in the model. In contrast, the hierarchical approach shows a tighter cloud of points with fewer extreme predictions and more stable behavior across AADT levels, denoting that extreme predictions are pulled toward the mean.

Figure 1: Literature specification and hierarchical framework fitted values against AADT for the Washington dataset

The results for Queensland Heavy Vehicles are shown in Table 2. The specification used as benchmark is a model fitted through an iterative search, given that the specification found in the literature is a case-control of the dataset used for the analysis. The variables used are the Average Annual Daily Travel (AADT), the total width of the segment (in meters), the number of lanes, an indicator function if the segment is a high-speed road segment, an indicator function if the segment is a medium-speed road segment, an indicator function if the road segment is urban single carriageway, an indicator function if the road segment is urban dual carriageway, an indicator function if the road segment is rural single carriageway, and an indicator function if the road segment is rural dual single carriageway, along with the speed limit (in km / hr.) and the multi-combination vehicular traffic.

The comparison between the benchmark model and the hierarchical specification shows a more nuanced trade-off between explanatory structure and predictive performance. Regarding parameters, both models are broadly consistent in direction in terms of multi-combination vehicular traffic, which shows a strong positive association with crashes in both cases, though the magnitude is smaller in the hierarchical model (0.92 vs 2.28), suggesting that part of its effect in the benchmark model may have been inflated due to omitted variables or lack of structure.

The rural single carriageway with high-speed limit indicator is also positive and highly significant in both models, with a larger effect in the hierarchical specification (1.38 vs 0.54), indicating improved identification of high-risk road types. A key structural difference is the replacement of the number of lanes, which is negative in the benchmark model, with total width, which is also negative in the hierarchical model. This is conceptually consistent, as both capture road capacity or space. The hierarchical model also introduces an additional level (β0) and a secondary rural indicator, capturing within group variability that is absent in the benchmark.

In terms of the goodness of fit, the benchmark model shows a lower BIC (642 vs 668) and a marginally higher log-likelihood (-302.32 vs -303.81). Regarding predictive performance, the hierarchical model improves MSE (0.059 vs 0.071) and RMSE (0.243 vs 0.267), along with a better MAE (0.087 vs 0.092).

Overall, the hierarchical model’s smaller coefficient for heavy vehicles may reflect a more realistic marginal effect once heterogeneity is accounted for. Finally, while the benchmark model does not include random parameters, the hierarchical model captures structured variability through two variance components (first and sub-level), though with relatively small magnitudes and only marginal statistical significance, suggesting that heterogeneity exists but is not pronounced. As a result, the hierarchical model provides a more theoretically grounded specification with gains in average predictive accuracy and similar fit, though the benefits of the hierarchical approach may depend on the complexity and heterogeneity of the underlying data in this case.

Table 2: Specification between complete pooling and hierarchical approach for the Heavy Vehicles QLD dataset.

| Benchmark |  |  | Specification found through the hierarchical approach |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Estimate | p-value | Parameter | Estimate | p-value |
| α0 | -12.3962 | <0.0001*** | α0 | -10.8590 | <0.0001*** |
| Number of lanes | -1.2646 | <0.0001*** | Total width | -0.2758 | 0.0118** |
| 1 if the road segment is rural single carriageway with high-speed limit, 0 otherwise | 0.5442 | <0.0001*** | 1 if the road segment is rural single carriageway with high-speed limit, 0 otherwise | 1.3780 | 0.0002*** |
| Multi-combination vehicular traffic | 2.2823 | <0.0001*** | Multi-combination vehicular traffic | 0.9219 | <0.0001*** |
|  |  |  | β0 | 2.1655 | <0.0001*** |
|  |  |  | 1 if the road segment is rural single carriageway, 0 otherwise | 0.1179 | 0.0687* |
| Random parameters |  |  |  |  |  |
| - | - | - | Std. Deviation (first level) | 0.0931 | 0.0803* |
| - | - | - | Std. Deviation (sub level) | 0.1915 | 0.0572* |
| Goodness of fit |  |  |  |  |  |
| BIC | 642.30 |  | 667.80 |  |  |
| Log-Likelihood | -302.34 |  | -303.81 |  |  |
| MSE | 0.071 |  | 0.059 |  |  |
| MAE | 0.092 |  | 0.087 |  |  |
| RMSE | 0.267 |  | 0.243 |  |  |

Figure 2 shows the scatterplot for fitted values of both approaches against the AADT in the QLD heavy vehicles dataset. In contrast to what is shown in Figure 1, the QLD heavy vehicles dataset exhibits a highly skewed distribution with a strong concentration near-zero predictions and a limited number of extreme values. While both models show a relatively weak dependence on AADT, the benchmark model generates substantial dispersion and more extreme predictions in the mid-range of exposure. The hierarchical model, although not drastically reducing overall variability, produces a more structured pattern of fitted values, suggesting the model captures latent group-heterogeneity rather than simply smoothing predictions.

Overall, these results suggest that, in settings with lower or more structured heterogeneity, the hierarchical approach provides benefits in terms of stability rather than large improvement in predictive accuracy.

Figure 2: Analyst specification and hierarchical framework fitted values against AADT for the QLD Heavy Vehicles dataset

The specification for the Maine dataset considers a set of design features and contextual variables. Specifically, an indicator function for the presence of curve, the posted speed limit, the left and right shoulders width, the number lanes, the number of days with precipitation, the number of days with temperature above 32°F, and an indicator function if the season is winter, the segment length (in miles), and the monthly AADT (MADT).

The comparison between the specifications found in the literature and the hierarchical approach is shown in Table 3. The benchmark is provided by (Islam, 2023). In the benchmark model, key exposure-related variables (e.g., MADT and segment length) show strong positive effects, as expected. Furthermore, speed limit shows a positive estimate, denoting an increase in crash risk, while shoulder width has a negative (protective) effect. However, environmental variables such as days with precipitation above 1 inch and days with temperature below 32°F are not statistically significant in the fixed part of the model, yet appear as significant random parameters, suggesting unobserved heterogeneity that the model cannot fully structure.

In contrast, the hierarchical approach integrates these environmental factors directly into the structure, where both precipitation and temperature-related variables become highly significant and positively associated with crashes. The inclusion of additional structure, such as the second-level intercept and a seasonal indicator, further enhances interpretability by explicitly modeling heterogeneity. In terms of the estimates, the hierarchical model maintains intuitive signs: higher speed limits increase risk, wider shoulders reduce crashes, and adverse weather conditions increase crash risks, all consistent with theory of road safety. Regarding the goodness of fit and predictive accuracy, the hierarchical model shows significant improvements, reducing MSE from 10.189 to 0.241, MAE from 1.275 to 0.170, and RMSE from 3.192 to 0.491, representing an order-of-magnitude gain in predictive performance.

Although the BIC and log-likelihood are not reported for the benchmark, the magnitude of the error reduction clearly favors the hierarchical approach, capturing structured heterogeneity through variance components for the first and sub-levels, both statistically significant.

Table 3: Specification between complete pooling from the literature and the hierarchical approach for the Maine rural dataset.

| Specification provided by (Islam, 2023) |  |  | Specification found through the hierarchical approach |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Estimate | p-value | Parameter | Estimate | p-value |
| α0 | -10.360 | <0.050** | α0 | -57.2378 | <0.0001*** |
| MADT | 0.805 | <0.050** | Speed limit | 0.4949 | <0.0001*** |
| Segment length | 0.955 | <0.050** | Right shoulder width | -0.0234 | <0.0001*** |
| Speed limit | 0.030 | <0.050** | Days with precipitation | 0.0440 | <0.0001*** |
| Shoulder width | -0.165 | <0.050** | Days with temperature above 32°F | 0.0455 | <0.0001*** |
| 1 if the segment has a curve, 0 otherwise | 0.204 | <0.050** | β0 | 7.5549 | <0.0001*** |
|  |  |  | 1 if the season is winter, 0 otherwise | 0.2350 | 0.070* |
| Random parameters |  |  |  |  |  |
| Days with precipitation > 1.0 (in) | 0.199 | <0.050** | Std. Deviation (first level) | 0.7208 | <0.0001*** |
| Days with temperature <32°F | 0.189 | <0.050** | Std. Deviation (sub level) | 0.1736 | 0.0024*** |
| Goodness of fit |  |  |  |  |  |
| BIC | - |  | 17,321.24 |  |  |
| Log-Likelihood | - |  | -8613.19 |  |  |
| MSE | 10.189 |  | 0.241 |  |  |
| MAE | 1.275 |  | 0.170 |  |  |
| RMSE | 3.192 |  | 0.491 |  |  |

The comparison of fitted values against the exposure term (AADT) for the literature specification and the hierarchical approach is shown in Figure 3. It is worth noting that the Maine dataset provides a different behavior compared to the other cases analyzed. While the literature-based model shows substantial dispersion and produces extreme values, which is particularly notable at high exposure levels, the hierarchical model yields highly concentrated predictions with minimal variation across the range of exposure. This pattern suggests a strong shrinkage effect, where the hierarchical model regularizes predictions toward the global mean. Although this behavior effectively mitigates the presence of extreme predictions, it may also indicate reduced sensitivity to traffic exposure, along with a potential underestimation of variability.

Figure 3: Literature specification and hierarchical framework fitted values against AADT for the Maine dataset

Table 4 shows the summary of the fitted values of the hierarchical approach and the benchmark models. Overall, the hierarchical approach provides fitted crash distributions closer to the empirical distributions compared to the benchmark models. The improvements were particularly pronounced for the Maine dataset, where the benchmark model substantially overestimated both average crash frequency and upper-tail observations (Q3 and maximum values). By incorporating group-level heterogeneity through partial pooling, the hierarchical framework produced more realistic values while preserving the observed distributional characteristics of the data.

Table 4: Comparison of observed and fitted crash count distributions for benchmark and hierarchical models

|  | Washington |  |  |  |  | QLD Heavy Vehicles |  |  |  | Maine |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Actual values | Fitted benchmark | Fitted Hierarchical |  |  | Actual values | Fitted benchmark |  |  | Fitted Hierarchical | Actual values | Fitted benchmark | Fitted Hierarchical |  |
| Min |  |  | 0.00 | 0.04 | 0.00 |  |  | 0.00 | 0.00 |  |  | 0.00 | 0.00 | 0.01 | 0.00 |  |
| Q1 |  |  | 4.00 | 5.69 | 5.81 |  |  | 0.00 | 0.00 |  |  | 0.00 | 0.00 | 0.38 | 0.00 |  |
| Mean |  |  | 10.00 | 8.65 | 9.53 |  |  | 0.05 | 0.06 |  |  | 0.07 | 0.06 | 1.32 | 0.11 |  |
| Q3 |  |  | 20.00 | 14.23 | 16.00 |  |  | 0.00 | 0.03 |  |  | 0.05 | 0.00 | 1.38 | 0.03 |  |
| Max |  |  | 140.0 | 155.5 | 130.04 |  |  | 3.00 | 3.74 |  |  | 2.99 | 5.00 | 10.55 | 5.30 |  |

CMF Derivation

As stated above, a key advantage of the hierarchical modeling framework analyzed in this study is its ability to derive CMFs directly from the estimated model in a unified and consistent manner as shown in Eq. (43). Unlike the traditional formulation proposed by the HSM, where CMFs are introduced as multiplicative adjustment factors applied to a baseline SPF previously developed, the hierarchical approach embeds treatment effects, exposure, and contextual factors within a single data-generating process.

For the Washington experiment, the specification is shown in Equation (53) with the estimates shown in Table 1.

Np=exp-10.3774-0.0534Xwidth per lane+0.0954Xcurves+0.1014Xtangent length+0.4192Xinterchanges+0.0769Xmax. grade diff-0.4338Xshoulder widthAADTexp5.3944-0.0769Xslope flat     (53)

In this sense, the CMFs for the baseline propensity are:

- CMFwidth per lane=exp-0.0534=0.948, denoting that each unit increase (in meters) in the total width of the road segment decreases crashes by about 5.2%.

- CMFcurves=exp0.0954=1.10, showing that each additional curve in the road segment increases crashes by about 10%.

- CMFtangent length=exp0.1014=1.106, which shows that increasing the tangent length (in miles) in one-unit increases crashes by about 10.6%.

- CMFgrade difference=exp0.0769=1.079, showing that increasing the grade difference in the segments by one-unit (in %) increases crashes by about 7.9%.

- CMFshoulder width=exp-0.4338=0.648, yielding a decrease of about 33.2% per one-unit increase (in meters) on the shoulder width in the road segments.

Note that the AADT does not have unit-elasticity like traditional models. Instead, it enters the model with as a function with its own parameter and the effect of the variable for flat slope segments. Furthermore, note that the effect of flat slope segments on crashes is not constant, instead it depends on the exposure term as it enters in the hierarchical structure of the model. In this sense, let’s consider N1 if Xslope flat=1 and N0 if Xslope flat=0, we can write the following for the slope:

N1∝(AADT)exp⁡(5.3944-0.0769)

And:

N0∝(AADT)exp⁡(5.3944)

Note that the ∝ mark denotes the proportional effect of the expected crashes considering the exposure levels, along with a constant k that is implicitly expressed in both expressions. This constant represents the effect of other features, specifically the baseline effects in the analysis. Later, taking the ratio of both expressions, it yields the following:

CMFAADT=N1N0=(AADT)exp5.3944-0.0794-exp⁡(5.3944)

Being an expression dependent on the exposure term. Furthermore, at low traffic levels there is a small effect on the slope, while for high exposure there is a larger effect, whose behavior depends on the level of traffic for the road segment to analyze. This implies that the safety effect of flat slope varies across traffic conditions, reducing the sensitivity of crash frequency to increases in exposure in models with fixed elasticities.

In the QLD Heavy Vehicles dataset, the specification is shown in Equation (54) taking the estimates from Table 2.

Np=exp-10.859-0.2758Xtotal width+1.378Xrural single high speed+0.9219Xmulti-combination vehicular traffic AADTexp⁡(2.1655+0.1179Xrural single carriageway)  (54)

Similarly, the CMFs derived from the specification in Eq. (54) are the following:

- CMFtotal width=exp-0.2758=0.758, denoting that increasing the total width of the road segment in one-unit (in meters) decreases crashes by about 24.2%.

- CMFrural single high speed=exp1.3780=3.966, showing that rural road segments with high-speed limit have about four times crashes compared to the baseline.

In this specification the AADT has a variable effect on crashes. Similarly, rural road segments with single carriageways have a dynamic effect on crashes as it depends on the exposure term. Importantly, not all estimates should be interpreted as CMFs in the baseline structure. In this sense, the multi-combination vehicular traffic is a contextual factor, not treatment, so it is better interpreted as risk modifier and not a deployable CMF.

Similar to the previous example, let’s consider N1=1 if Xrural single carriageway=1 and N0=0 if Xrural single carriageway=0 to write the following:

CMFAADT=(AADT)exp2.1655+0.1179-exp⁡(2.1655)

Which denotes an expression dependent on the exposure levels. As a result, the CMF is traffic-dependent and represents a functional form where the CMF increases with AADT, indicating that rural single carriageway segments experience a stronger growth in crash frequency as traffic volume increases.

Finally, in the case of the Maine dataset, the specification yielded from the estimates shown in Table 3 is shown in Equation (55).

Np=exp-57.2378+0.4949Xspeed limit-0.0234Xright shoulder width+0.044Xdays with precipitation+0.0455Xdays with temperature<32°FAADTexp7.5549+0.2350Xif the season is winter (55)

Which yields the following CMFs:

- CMFspeed limit=exp0.4949=1.64, showing that increasing the posted speed limit increases crashes by about 64%.

- CMFright shoulder width=exp-0.0234=0.977, denoting that increasing the shoulder width (in meters) by one-unit, decreases crashes by about 2.3%.

Other variables, such as the days with precipitation and days with temperature below 32°F work as baseline risk modifiers and contextual factors and non-deployable CMFs. Furthermore, in this specification the exposure affects the effect on the crashes if the season is winter, as it enters as a function of such variable in the log-form of the specification shown in Eq. (55).

Although it is not a CMF, we can write the effect of the season in the hierarchical structure in following expression:

CMFAADT=(AADT)exp7.5549+0.2350-exp⁡(7.5549)

Denoting the season works as a risk modifier depending on the exposure of the road segments in the model specification.

Discussions

This study contributes to the literature on road safety by providing properties and advantages derived from hierarchical model’s framework, along with comparison to traditional methods. The setting includes the integration of exposure, geometric design features, and contextual factors in a single unified structure. Unlike traditional approaches based on the multiplicative formulation provided by the HSM, where exposure is treated externally and with a fixed elasticity, the hierarchical framework incorporates exposure directly into the data-generating process, allowing its effect to vary across diverse contexts. This distinction has important implications for both statistical inference and the interpretation of crash modification effects of features.

A central contribution of this work lies in the explicit separation between baseline crash propensity and traffic-response behavior. Conventional methods of SPF formulations, exposure is introduced through offsets or multiplicative scaling, implicitly assuming constant elasticity of crashes with respect to traffic volume. However, empirical and theoretical studies have consistently shown that relationships between traffic flow, crash frequency, and crash severity are non-linear, often exhibiting a saturation effect (Deflorio, 2022; Fiorentini, 2025). By modelling exposure as a varying coefficient within a hierarchical structure, the proposed approach allows traffic elasticity to depend on contextual and geometric characteristics, providing a more realistic representation of the data generation process in crash mechanisms (Munira, 2020). Furthermore, this result not only aligns with existing theoretical insights but also enhances the interpretability of the resulting SPF.

Regarding methodological perspectives, the hierarchical framework addresses a fundamental limitation of traditional regression-based models in relation to the treatment of heterogeneity. Road safety data are inherently structured, with observations nested within segments, functional classes, environmental contexts or urban conditions (Mannering, 2020). Ignoring such structure, as in complete pooling models, can lead to biased estimates due to omitted group-level effects (Mitra, 2012). Conversely, estimating separate models for each group (no pooling) often results in unstable estimates, particularly when group sizes are small. In this sense, the hierarchical approach provides a principled compromise through partial pooling effects, allowing information to be shared across groups while preserving relevant heterogeneity. This property is closely related to the EB framework, a method widely used in road safety analysis, which reinforces the theoretical consistency of the proposed model (Fawcett, 2017).

Another important implication of hierarchical specification is its ability to mitigate omitted variable bias. Traditional methods of CMF development often fail to properly account for exposure and contextual factors, which can result in confounding, where the estimated effects of geometric features absorb the influence of unobserved variables (Hauer, 2010). This can lead to counterintuitive or misleading interpretation of estimates, such as attributing increased crash risk to design features that are merely correlated with higher traffic volumes (Fiorentini, 2025). By incorporating exposure and contextual variables within a unified model and allowing for cross-level interactions, the hierarchical framework reduces the risk of such biases and produces more behavioral estimates.

The empirical results further illustrate how these theoretical advantages are translated into practice, although the magnitude of the benefits of the hierarchical approach depends on the characteristics of the data. In settings with substantial heterogeneity, the hierarchical model improves predictive performance and stabilizes estimates by reducing extreme predictions. In more homogeneous or structured datasets, the gains are more modest and primarily relate to improved interpretability and robustness. In this sense, the results also highlight the importance of carefully specifying the variance structure and assessing the balance between flexibility and regularization.

Beyond statistical considerations, the proposed framework has important implications for the development of SPF and CMF. By embedding CMFs within a hierarchical model, the approach enables the derivation of traffic-sensitive treatment effects, where the impact of a design intervention may vary with exposure levels and contextual conditions. In practical terms, this means that the effectiveness of a given treatment may differ across road segments depending on traffic intensity and environmental factors, which is critical for informed decision-making in road safety management.

Overall, the hierarchical modelling framework offers a flexible and theoretically grounded alternative to conventional methods in road safety analysis. It enhances the representation of crash generation processes, improves the robustness of statistical inference, and provides a coherent framework for integrating exposure, design, and context into a single specification. At the same time, the findings emphasize that the effectiveness of the approach is context-dependent, and its application should be guided by the structure and quality of the available data. Future research could extend this framework by incorporating spatial dependence, temporal dynamics, or more complex hierarchical structures, further improving its applicability in real-world settings.

Implications

The findings of this study have several important implications for practitioners involved in road safety management, infrastructure design, geometric design, and policy evaluation. First, the results highlight the limitations of relying on traditional crash prediction models that assume fixed relationships between traffic exposure and crash frequency. In practice, this implies that standard methods for developing SPFs and CMFs may not adequately capture the true safety effects of design interventions across different traffic and environmental conditions. The hierarchical framework analyzed in this study provides a more flexible alternative, allowing practitioners to account for context-dependent effects, thereby improving the reliability of safety assessments (Wang, 2025).

A key implication of the framework is the ability to derive traffic-sensitive CMFs, where the effectiveness of a given treatment varies with exposure levels. From a policy perspective, this means that single CMF values may not be universally applicable across all road segments, especially for varying exposure levels (Gan, 2022). By incorporating the exposure directly into the model and allowing its elasticity to vary, practitioners can develop more targeted and context-specific safety interventions, leading to more efficient allocation of resources.

Furthermore, the hierarchical approach enhances decision-making by reducing the influence of extreme or unstable predictions, which are commonly observed in traditional models, particularly in datasets with high variability. For practitioners, this translates into more stable and robust estimates of crash risk, which is critical when prioritizing safety improvements or evaluating the potential impact of infrastructure changes (Jiaxu Jin, 2025). In particular, the reduction of extreme predictions can prevent the overestimation of risk in certain locations with specific contexts, thereby avoiding inefficient or misdirect investments.

Another important implication relates to the treatment of heterogeneity across road segments. The results show that road safety is not influenced by a single homogeneous process, but rather by multiple underlying mechanisms influenced by geometry, environment, and traffic conditions. In this sense, the hierarchical framework allows practitioners to explicitly model this heterogeneity, providing insights into how risk varies across different contexts. This is especially relevant for agencies managing diverse road networks, where policies based on average effects may fail to address location-specific risks if there are issues with the specification and framework.

Finally, the framework aligns directly with existing practices based on EB methods, which are widely used in road safety analysis. By extending these principles within a unified hierarchical structure, the approach provides a natural and theoretically grounded extension of current methodologies, facilitating its adoption in practice. This compatibility suggests that transportation agencies can integrate hierarchical modelling into existing workflows with relatively minor adjustments, while benefiting from improved statistical efficiency and interpretability.

Overall, the hierarchical modeling framework offers practitioners a more flexible, robust, and context-sensitive tool for road safety analysis, enabling better-informed decisions and more effective safety interventions. However, its successful implementation requires careful consideration of data characteristics, model specification, and the balance between enough flexibility and regularization.

Conclusion

This study develops and evaluates a hierarchical modelling framework for crash frequency analysis integrating exposure, geometric design features, and contextual factors within a unified specification aligned with recommendations provided by the HSM. Unlike traditional approaches based on multiplicative structures, where exposure is treated externally with fixed elasticity, the proposed framework incorporates exposure directly into the data generation process, allowing its effect to vary across contexts.

The results show that the hierarchical approach provides consistent improvements in statistical performance and interpretability. Across the Washington, QLD Heavy Vehicles, and Maine datasets, the model reduces prediction error, stabilizes fitted values, and mitigates extreme predictions commonly observed in traditional specifications. These improvements are particularly pronounced in settings with substantial heterogeneity, where the hierarchical structure effectively captures latent group-level variability though partial pooling.

A key contribution of this study is the explicit separation between baseline crash propensity and traffic-response behaviour. By modelling exposure as a varying coefficient, the framework relaxes the restrictive assumption of constant elasticity between traffic volume and crash frequency. This setting allows the model to capture non-linear and context-dependent relationships between the exposure and safety outcomes, aligning with theoretical and empirical findings in road safety literature.

Furthermore, the hierarchical framework enables the direct derivation of CMFs within a one-stage modelling process. Unlike traditional approaches that estimate CMFs as fixed multiplicative adjustments applied to a baseline SPF, the proposed model embeds treatment effects within the same structure as exposure and contextual variables. As a result, CMFs can be interpreted as functions rather than constants, allowing treatment effects to vary with traffic levels and environmental conditions. This provides a more flexible and behaviourally consistent representation of safety effects, specifically for design interventions whose impact are exposure dependent.

The empirical results also highlight the role of contextual variables in reducing omitted variable bias. By explicitly incorporating environmental and operational factors into the model, the hierarchical specification prevents geometric design effects from absorbing the influence of exposure and other confounding variables. This leads to more stable and theoretically consistent estimates, improving both inference and policy relevance.

Overall, the hierarchical CMF framework offers a statistically efficient and conceptually consistent alternative to conventional methods in road safety analysis. It enhances the representation of crash generation mechanisms, improves predictive reliability, and provides a unified approach for developing SPFs and CMFs. Furthermore, the results indicate that the magnitude of the advantages depends on the structure and heterogeneity of the data, with stronger gains observed in more complex and heterogeneous settings.

Future research could extend the framework by incorporating spatial dependence, temporal dynamics, or more flexible non-linear specifications. These extensions would further improve the ability of hierarchical models to capture the complex mechanisms underlying crash occurrence and support more accurate and context-sensitive safety evaluations.

CMF=exp⁡(αjΔx)

CMF(AADT)=exp⁡(αj+γjlog⁡(AADT))Δx

# References

AASHTO. (2010). Transportation Research Board. Task Force on Development of the Highway Safety Manual, & Transportation Officials. Joint Task Force on the Highway Safety Manual. (Vol. 1).

Bahar, G. &. (2014). User’s guide to develop highway safety manual safety performance function calibration factors. National Cooperative Highway Research Program, Transportation Research Board.

Butt, M. S. (2025). A literature review: AI models for road safety for prediction of crash frequency and severity. Discover Civil Engineering, 2(1).

Chen, Y. &. (2014). Methodology to develop crash modification functions for road safety treatments with fully specified and hierarchical models. Accident Analysis & Prevention, 70, 131-139.

Deflorio, F. &. (2022). Safety systems and vehicle generations: Analysis of accident and travel data collected using event data recorders. Journal of Transportation Safety & Security, 14(8), 1307-1332.

Elvik, R. (2009). The non-linearity of risk and the promotion of environmentally sustainable transport. Accident Analysis & Prevention, 41(4), 849-855.

Elvik, R. (2013). International transferability of accident modification functions for horizontal curves. Accident Analysis & Prevention, 59, 487-496.

Elvik, R. H. (2009). Basic concepts of road safety research. The Handbook of road safety measures, 81-98.

Elvik, R. H. (2009). The handbook of road safety measures. Emerald Group Publishing Limited.

Fawcett, L. T. (2017). A novel Bayesian hierarchical model for road safety hotspot prediction. Accident Analysis & Prevention, 99, 262-271.

Fiorentini, N. &. (2025). Developing safety performance functions to inform transport policies on urban two-lane roads. Research in Transportation Business & Management, 60, 101357.

Gan, X. W. (2022). Spatial-temporal varying coefficient model for lane-changing behavior in work zone merging areas. Journal of Transportation Safety & Security, 14(6), 949-972.

Gayah, V. V. (2021). Estimating safety performance functions for two-lane rural roads using an alternative functional form for traffic volume. Accident Analysis & Prevention, 157.

Gelman, A. &. (2007). Data analysis using regression and multilevel/hierarchical models. Cambridge university press.

German, A. C. (2025). Bayesian Data Analysis.

Hauer, E. (2010). Cause, effect and regression in road safety: A case study. Accident Analysis & Prevention, 42(4), 1128-1135.

Hodson, T. O. (2021). Mean squared error, deconstructed. Journal of Advances in Modeling Earth Systems, 13(12).

Huang, H. &.-A. (2010). Multilevel data and Bayesian analysis in traffic safety. Accident Analysis & Prevention, 42(6), 1556-1565.

Huang, H. C. (2008). Severity of driver injury and vehicle damage in traffic crashes at intersections: a Bayesian hierarchical analysis. Accident Analysis & Prevention, 40(1).

Islam, A. M. (2023). Grouped Random Parameters Negative Binomial-Lindley for accounting unobserved heterogeneity in crash data with preponderant zero observations. Analytic methods in accident research, 37, 100255.

Jiaxu Jin, H. F. (2025). Road safety measurement with reliability using an advanced hybrid decision model. Scientific Reports, 15(1).

Jones, A. P. (2003). The use of multilevel models for the prediction of road accident outcomes. Accident Analysis & Prevention, 35(1), 59-69.

Kim, D. G. (2007). Modeling crash outcome probabilities at rural intersections: Application of hierarchical binomial logistic models. Accident Analysis & Prevention, 39(1), 125-134.

Lord, D. &. (2010). The statistical analysis of crash-frequency data: A review and assessment of methodological alternatives. Transportation research part A: policy and practice, 44(5), 291-305.

Mannering, F. B.-A. (2020). Big data, traditional data and the tradeoffs between prediction and causality in highway-safety analysis. Analytic methods in accident research, 25, 100-113.

Mitra, S. &. (2012). On the significance of omitted variables in intersection crash modeling. Accident Analysis & Prevention, 49, 439-448.

Munira, S. &. (2020). A geographically weighted regression model to examine the spatial variation of the socioeconomic and land-use factors associated with Strava bike activity in Austin, Texas. Journal of Transport Geography, 88.

Park, H. C. (2019). Injury severity of bus–pedestrian crashes in South Korea considering the effects of regional and company factors. Sustainability, 11(11), 3169.

Pervaz, S. B. (2022). Integrating macro and micro level crash frequency models considering spatial heterogeneity and random effects. Analytic methods in accident research, 36. Retrieved from https://doi.org/10.1016/j.amar.2022.100238

Simon Washington, M. G. (2020). Statistical and Econometric Methods for Transportation Data Analysis (Vol. 3). Chapman and Hall/CRC.

Vayalamkuzhi, P. &. (2016). Influence of geometric design characteristics on safety under heterogeneous traffic flow. Journal of traffic and transportation engineering (English edition), 3(6), 559-570.

Venkataraman, N. S. (2014). Extending the Highway Safety Manual (HSM) framework for traffic safety performance evaluation. Safety science, 64, 146-154.

Wagner, P. H. (2021). Observations on the relationship between crash frequency and traffic flow. Safety, 71(3), 3.

Wali, B. K. (2018). Development of safety performance functions: Incorporating unobserved heterogeneity and functional form analysis. Transportation research record, 2672(30), 9-20.

Wang, C. F. (2025). Data-Driven Road Traffic Safety Modeling: A Comprehensive Literature Review. Applied Sciences, 16(1).

Wu, L. &. (2017). Examining the influence of link function misspecification in conventional regression models for developing crash modification factors. Accident Analysis & Prevention, 102, 123-135.