| Potential methods of developing crash models # Development of Crash Predictive Models for Road Design - Scoping Project |  |
| --- | --- |
| Submitted To: Queensland Department of Transport and Main Roads (TMR) |  |
| Prepared By: Alexander Paz Ph.D., FIEAust., CPEng, NER., RPEQ, P.E. Professor and Transport Main Roads Chair E mail: alexander.paz@qut.edu.au Zeke Ahern, Ph.D. QUT Postdoctoral Research Fellow |  |

Gonzalo Andrés Ríos Vásquez

PhD Student

Table of Contents

1.	Introduction	4

2.	Study design and mechanism to control for confounding factors	4

2.1.	Observational before and after studies	5

2.2.	Comparison group method	5

2.3.	Yoked comparison method	6

2.4.	Naïve approach	6

2.5.	Empirical bayes method	6

2.6.	Full Bayes method	7

2.7.	Cross-sectional studies	7

3.	Analysis and modelling methods	8

3.1.	SPFs, CMFs and CMFunctions	8

3.1.1.	CMFs in Australia	10

3.1.2.	Other relevant studies from Australia	12

3.2.	Fully specified models	12

3.3.	CMFunctions developed through a hierarchical model	13

4.	Empirical findings from literature	14

5.	Development and implementation roadmap	15

5.1.	Network definition and scope	16

5.2.	Segmentation analyses	16

5.3.	Model specification and SPF estimation	17

5.4.	Calibration and transferability assessments	18

5.5.	Sample size and number of sites	18

6.	Experimental and illustrative design of SPF and CMFs	19

6.1.	Washington dataset	19

7.	Discussions	22

7.1.	Alignment with HSM guidelines and current practice	22

7.2.	Implications for future research and policy development	23

8.	Conclusions	24

References	24

# Introduction

The Queensland Department of Transport and Main Roads (TMR) is interested in methods available for predicting the effect of geometric characteristics on crash rates and the potential cost of developing them. This is particularly motivated by the need to analyse the effect of Design Exceptions (DEs), which are defined as values of geometric characteristics (parameters) that fall outside the range specified in the Austroads Guide to Road Design and Road Planning and Design Manual (Austroads 2025). These DEs require a risk assessment for approval.  When upgrading existing roads, it is often unaffordable to upgrade all DEs to values by meeting these guidelines. At the same time, road designers are responsible for ensuring DEs that are retained provide an acceptable level of safety. Without a quantitative method and data-driven insights, engineers often rely on subjective assessments based on risk management frameworks or professional experience.

There is increasing interest in analysis methods involving statistically rigorous models that facilitate reproducibility, and causal interpretation. In addition, these methods should provide the ability to capture the marginal safety effects of individual geometric design elements while controlling traffic exposure, unobserved heterogeneity, and spatial and temporal dependence. Contemporary road-safety research has predominantly relied on Safety Performance Functions (SPFs), typically specified within count-data modelling frameworks such as Poisson, Negative Binomial, and hierarchical extensions, to estimate expected crash frequencies under varying geometric and operational conditions. However, the suitability and inferential validity of these methods vary substantially across applications, particularly when assessing isolated DEs on existing roadway networks, where limited variation, selection bias, and confounding pose significant econometric challenges.

Consequently, a systematic and methodologically grounded assessment of these modelling frameworks is required to support evidence-based decision-making, allowing practitioners to explicitly trade off expected safety outcomes against economic, operational, and geometric design constraints. Such assessments provide guidance on when simplified predictive formulations (e.g., calibrated base SPFs or aggregate CMFs) are statistically adequate, when more advanced modelling frameworks are empirically justified, and how estimation and prediction uncertainty should be formally quantified and communicated within DE risk assessment processes.

Hence, the aim of this project is to review and assess quantitative methods available for predicting the effect of geometric characteristics on crash rates. The following sections discuss the most common and sound methods used in practice for the estimation of the effects of changes to geometric characteristics, and other relevant countermeasures, on road safety.

# Study design and mechanism to control confounding factors

The estimation of the safety effects of roadway characteristics, treatments, weather and environmental conditions, and volumes requires statistical and evidence-based analyses to disentangle the intricate interactions involving the various factors contributing to crashes. There are several methods proposed in literature and practice to predict the safety effects of specific roadway characteristics and treatments. Considering the study design and mechanism to control for confounding factors, the most common methods are observational Before and After (BA) and cross-sectional studies. Other less common methods include randomized controlled trials, case–control studies, cohort studies (e.g., monitoring drivers over time), and naturalistic experiments (e.g., introduction of random breath testing in one state versus another).

## Observational before and after studies

BA studies are typically preferred over cross-sectional studies because they enable a longitudinal analysis considering actual changes within a single dataset over time from the ‘before’ to the ‘after’ condition (Sawalha and Sayed 2001). Therefore, BA studies directly account for unobserved site-specific characteristics, because each site serves as its own control. BA studies provide a direct measure of treatment effect because they capture changes in crashes after intervention. Additionally, results are intuitive and policy-friendly making them straightforward to communicate to stakeholders (e.g., crashes fell by X% after treatment). However, special consideration should be given to other changes over the period of analysis, such as volumes and regulations. In particular, it is important to control for regression-to-the-mean bias using methods such as the Empirical Bayes, as recommended by the Highway Safety Manual (AASHTO, 2010). Furthermore, BA studies are not feasible for all types of roadway characteristics such as curves because changing the alignment of curves is both costly and time-consuming. More importantly, typically when an alignment is improved, other roadway characteristics such as pavement type, lane width, side slope, and superelevation are also changed.

This limitation gives rise to confounding temporal effects that are not adequately addressed by BA study designs. Observed changes in crash frequency over time may be driven by factors exogenous to the treatment itself, including traffic growth or decline, land-use dynamics, shifts in travel behaviour, and secular improvements in vehicle safety technologies. Moreover, BA studies offer limited capacity to properly control for changes in traffic exposure. Because crash counts are highly sensitive to exposure levels, naïve BA comparisons that fail to adjust for variations in traffic volume or operating conditions across periods can lead to biased and inconsistent estimates of treatment-related safety effects (Elvik, 2002).

Traditional analytical approaches used in observational BA safety studies include the naïve (or simple) BA approach, comparison group (CG) methods, yoked comparison (YC) designs, Empirical Bayes (EB) techniques, and fully Bayesian (FB) hierarchical modelling frameworks.

## Comparison group method

The CG approach estimates the number of crashes that would have occurred at treated sites in the absence of the treatment. A set of comparison sites is selected to proxy external factors that may influence temporal changes in crash frequency, such as regional trends in traffic demand, land-use patterns, or vehicle safety (Mountain, et al., 1992). The CG approach relies on two key assumptions: (i) unobserved factors affecting safety evolve similarly over time at both treated and comparison sites, and (ii) any changes in observable and unobservable determinants of safety are identical across treated and comparison groups (Shen, 2007). Under these assumptions, the expected crash frequency at a treated site in the after period, had the treatment not been implemented, denoted Nsite, after is estimated through Nsite, after=Nsite,before×rbefore,after. Where Nsite, before is the observed crash frequency in the before period and rbefore, after is the ratio of after-to-before crash frequencies observed at the comparison sites. The corresponding CMF is then computed as the ratio of the observed crash frequency in the after period at the treated sites to the counterfactual expected crash frequency Nsite, after, representing the estimated safety effect attributable to the treatment.

## Yoked comparison method

The YC method can be viewed as a special case of the CG approach. In the YC framework, each treated site is paired with a single comparison site selected to closely match pre-treatment characteristics, typically in terms of traffic exposure, operational conditions, and geometric design features. A practical advantage of the YC method is its relatively low data requirement compared with CG designs. However, this parsimony also constitutes a potential limitation. When matching quality is inadequately assessed or when unobserved heterogeneity remains, the YC approach yields biased counterfactual estimates and lead to misleading inferences regarding treatment effects (Gross et al., 2010).

## Naïve approach

Under the naïve BA approach, the expected number of crashes in the after period is obtained by scaling the observed crash count in the before period by the ratio of the after-period duration to the before-period duration, implicitly assuming a stable underlying crash-generation process (Abdel-Aty et al., 2014). A fundamental limitation of this approach is its susceptibility to regression-to-the-mean bias, particularly when treatment sites are selected based on unusually high crash frequencies in the pre-treatment period. As a result, the naïve method systematically overestimates treatment effectiveness by attributing natural reversion toward long-run mean crash levels to the intervention itself. Empirical studies have showed that, even in the absence of any actual safety improvement, the naïve approach can incorrectly predict substantial crash reductions following hypothetical treatments, underscoring its inability to account for regression to the mean and other sources of temporal confounding (Lan et al., 2009).

## Empirical Bayes Method

The EB approach was introduced in road-safety analysis to address key limitations of the naïve, CG, and YC methods, most notably their failure to adequately control regression-to-the-mean bias (Hauer et al., 2002). The EB framework estimates the expected number of crashes as a weighted average of the observed crash counts at treated sites and the corresponding model-based predictions derived from SPFs. The weights are determined by the relative precision of the observed data and the prior mean, allowing the method to shrink extreme observations toward their expected values. By explicitly incorporating traffic exposure, temporal trends, and site-specific characteristics through the SPF, the EB method provides more reliable counterfactual estimates of crash frequency. Empirical evidence consistently shows that EB-based evaluations outperform traditional comparison-based approaches by simultaneously accounting for regression-to-the-mean, exposure effects, and unobserved heterogeneity (Ko et al., 2013; Persaud and Lyon, 2007).

## Full Bayes method

The FB approach is conceptually related to the EB framework, in that both methods rely on non-treated reference sites to construct counterfactual estimates while controlling for temporal trends and other influences unrelated to the treatment (Lan et al., 2009). The key distinction lies in the estimation of the crash prediction model. In the FB approach, the model parameters and site-specific crash frequencies are jointly estimated within a fully Bayesian hierarchical framework using data from both treated and reference sites in the pre-treatment period. In contrast, under the EB approach, the crash prediction model is typically estimated exclusively using data from reference sites, and the resulting predictions are subsequently combined with observed crash counts at treated sites through a shrinkage estimator (Sacchi and Sayed, 2015). As a result, the FB approach propagates parameter uncertainty more comprehensively and provides full posterior distributions of counterfactual crash frequencies, whereas EB relies on plug-in estimates from the reference-site model.

Considering some of the above limitations, recent studies have extended BA designs using causal inference methods and quasi-experimental frameworks such as Difference-in-Differences (DiD) (Zhu et al., 2025; Elvik, 2024; Hosking, et al., 2023), Synthetic Control (McCoy, 2025) which further mitigate confounding from unobserved time-varying effects

## Cross-sectional studies

Cross-sectional studies have the advantage that they do not require post-treatment data but rather use existing crash, volume, and roadway inventory data from multiple locations with and without the characteristics of interest. Cross-sectional studies are cost-effective because there is no need to track individual sites over time and large sample sizes are possible thereby improving statistical power. In addition, cross-sectional studies enable identification of broad relationships between roadway characteristics and crash outcomes. Hence, provided that the necessary data is available, cross-sectional studies enable the study of all DEs required by TMR including but not limited to the following road characteristics:

- Vertical crest curve radius

- Cross section width

- Lane width, shoulder width, wide centerline

- Horizontal curve radius

- Stopping sight distance (around horizontal curves and intersections)

- Intersection turn type

- Requirement for roadside barriers

- Adverse superelevation / crossfall

- Water film depth at horizontal curve transitions

- Motorcycles rub rail length

- Roadside barrier length and location

- Floodway design

Cross-sectional studies can quantify marginal effects of geometric design features, allowing researchers to estimate the independent (marginal) effect of individual geometric and operational variables, while controlling for exposure across road segments. Furthermore, when these studies are conducted using large and representative datasets, cross-sectional models can produce results that are transferable across similar roads or highways, supporting better strategic planning and providing insights beyond the specific context of application. Despite these advantages, it is important to mention limitations of cross-sectional studies. One of the most important issues with this approach is the vulnerability to omitted variable bias (unobserved confounding effects), which is mainly due to data availability issues. Furthermore, cross-sectional studies have a limited causal interpretation as they rely on observational data, yielding associational estimates rather than causal inference (Gooch et al., 2016).

# Analysis and modelling methods

From an analysis or modelling standpoint the most common methods used in research and practice to assess the effectiveness of treatments or improvements in roadway characteristics include: (i) Safety Performance Functions (SPFs) multiplied by crash modification factors or functions (CMFs or CMFunctions), (ii) fully specified models including multiple covariates (Lord et al., 2008), and (iii) hierarchical models (Chen and Persaud, 2014).

## SPFs, CMFs and CMFunctions

SPFs can only be applied using CMFs or CMFunctions to adjust predictions for conditions consistent with the underlying model characteristics. In theory, fully specified models do not require the use of CMFs because they include the factors that influence safety as well as their corresponding effects captured through coefficients. However, the estimation of fully specified models including all relevant factors and interactions is still a significant challenge. Therefore, the first edition of the HSM did not consider fully specified models. Instead, it recommends a crash prediction framework as follows:

Npredicted,x = NSPFx ⋅ (CMF1x ⋅ CMF2x ⋅ . . . ⋅ CMFyx) ⋅ Cx                                 (1)

where Npredicted,x is the predicted average crash frequency for a site type x for a specific year; NSPFx is the predicted average crash frequency determined for base conditions of the SPF developed for site type x; CMFyx are the CMFs or CMFunctions specified for SPFx, and Cx is a calibration factor to adjust SPF for local conditions for site type x. Hence, a CMF is only a multiplicative factor to estimate the effect on predicted crash frequency because of a change on a roadway characteristic and/or countermeasure (AASHTO, 2010). CMFs are only reliable when they are both methodologically and statistically valid (Harkey et al., 2008). However, many CMFs in literature and practice were developed through questionable approaches involving BA studies without considering regression-to-the-mean bias, and/or insufficient data (Sayed and de Leur, 2008). Therefore, many potential CMFs were excluded from the first HSM edition (Federal Highway Administration, 2011).

A fundamental aspect that is not addressed by current CMF applications is the multiplication or correlation effects of factors when they are applied alone versus along with others. That is, there is insufficient evidence that the real combined effect of multiple changes applied simultaneously can be properly estimated by multiplying the SPFs with the CMFs which were developed considering only one change. Another key challenge is whether a fixed CMF is sufficient or whether a CMFunction is required or more appropriate. Elvik (2009) provided evidence that it is better to show how the effect of a change varies as a function of one or more characteristics that influence its magnitude. That is, a CMFunction can enable the estimation of safety effects as a function of other factors thereby addressing cross-factor correlations and enhancing the analysis by providing additional information that can be used to improve outcomes and minimize crashes. Similarly, Hauer et al. (2012) argues about how the effects of strategies to improve road safety vary systematically. Further, CMFunctions are better than CMFs to describe the effect of a treatment considering multiple characteristics because the effectiveness of safety countermeasures is likely to be affected by the attributes of the treated sites. For example, Sacchi and Sayed (2014) showed that treatment effectiveness varies considerably with post-treatment time and treatment site covariates. Hence, ignoring these two factors can lead to inaccurate estimate of the safety treatment effectiveness and potentially misapplication of available safety project funds.

Developing CMFs requires assessing the quality of evaluation studies including capturing systematic patterns on the distribution of the estimated safety effects (Elvik, 2015). Furthermore, it is important to consider the heterogeneity among locations and the corresponding effect on safety outcomes (Sacchi and Sayed 2014). Elvik (2015) proposed guidelines for developing CMFunctions.

A key aspect during the development of CMFs and CMFunctions is the identification of independent variables of primary interest which are of two types:

1. Roadway characteristics associated with the safety treatment such as indicators of its quality or extent of use.

2. Context characteristics such as the type of traffic environment.

A common challenge during the development of CMFs and CMFunctions is the heteroscedasticity of the data where data points have different sampling variance. Data points with low and high number of crashes will have a greater and smaller variance, respectively. This is a common characteristic of most data sets used in meta-analysis of road safety evaluation and likely to be a problem for developing CMFs and CMFunctions. One of the main reasons for the presence of heteroscedasticity is the unobserved heterogeneity presented in the site-level effects. Heteroscedasticity can be detected by plotting residuals versus fitted values as graphical diagnostics, comparing residuals and exposure, and formal tests in statistics such as Breusch-Pagan or White tests. Addressing this issue depends mostly on whether heteroscedasticity is treated as a problem affecting inference or as a structural feature of the crash processes. Treatments of this issue include the use of robust standard errors (heteroscedasticity-consistent) that keep estimates unchanged but adjust standard errors to remain valid under heteroscedasticity. Often, applying transformations to the dependent variable is used to address heteroscedasticity. Some common approaches include logarithmic or power transformations. Other methods involve the use of advanced modelling approaches, such as modelling the mean-variance relationship in count models, heteroscedastic regression models, or the inclusion of random-parameter (mixed) models that allow coefficients to vary across sites and tackle unobserved heterogeneity.

Developing CMFs or CMFunctions using previous studies also requires dealing with several common challenges including (Elvik, 2015):

1. Previous studies are likely to have employed different designs and are of variable quality.

2. Sources of systematic variation in road safety effects cannot be identified.

3. There are sources of systematic variation in road safety effects that do not reveal a systematic pattern.

To address most of the above challenges during the development of CMFunctions, Elvik (2015) proposed and illustrated the use of ten methodological guidelines. These guidelines involve:

- Classify, code and select relevant studies that are used as the basis for developing CMFunctions.

- Determine the contribution of systematic variation to the overall variation in estimates of effect considering possible presence of publication bias and whether estimates of effect vary systematically among countries and/or over time.

- Identify independent variables for CMFunctions.

- Detect outlying data points.

- Identify the most parsimonious functional form for the CMFunctions.

- Determine if more than one function should be developed to capture trends and variations in a data set.

- Evaluate the quality of the CMFunctions using statistical analyses and criteria.

- Perform sensitivity analysis regarding the various decisions made for developing the CMFunctions.

- Assess heteroscedasticity and methods to address it.

- Establish a protocol for updating the CMFunctions.

Despite these guidelines, developing CMFs or CMFunctions continues to be a significant challenge even considering results and data from other jurisdictions. For example, using data from Florida and Ohio as well as SPFs from the Highway Safety Manual, Wang et al. (2016) showed that CMFs may be significantly different when SPFs from other states are applied. Therefore, CMFs would be biased if proper adjustments to the SPFs from other states are not performed. In addition, the most common sources of CMFs are the Highway Safety Manual, the Handbook of Road Safety Measures, and FHWA’s Crash Modification Factor Clearinghouse website (AASHTO, 2010; Elvik et al., 2009; FHWA, 2011). These sources include the CMFs and sometimes their standard error (ES). However, none provides estimates of the standard deviation of the CMF which is required to be able to make a correct assessment (Hauer et al., 2012).

### CMFs in Australia

A study by the National Asset Centre of Excellence (NACOE) found that Crash Reduction Factors (CRFs = (1- CMFs)·100) in literature were overwhelmingly general and mostly did not consider relevant aspects such as vehicle mode, traffic volume or speed (McHeim, 2020). The study produced an updated matrix of CRFs for TMR. The following jurisdictions in Australia were explicitly included in the study:

- Queensland – Transport and Main Roads (TMR)

- Victoria – VicRoads (Department of Transport)

- New South Wales —Transport for New South Wales (TfNSW)

- South Australia – Department of Planning, Transport and Infrastructure (DPTI)

- Western Australia – Main Roads Western Australia (MRWA)

- New Zealand — New Zealand Transport Authority (NZTA)

Highlights from the review include:

- It was not possible to compare all treatments specified by TMR with those from each of the road agencies because the corresponding CRFs were not available.

- The CRFs from TfNSW and TMR were the most similar.

- TfNSW disaggregated CRFs by crash type (road user movement codes) and speed zone (≤ 60km/h, 70–80 km/h, >80km/h).

- TMR, TfNSW and DPTI disaggregated CRFs by speed and crash type, MRWA by crash type only.

- VicRoads and NZTA tended to provide all casualty reduction percentages.

- Two-thirds (60 out of 90) of the CRFs from NZTA have a low confidence level or none was provided.

- None of the treatments provided a breakdown by road user type, traffic volumes or vehicle type. The exceptions were those that applied only to one road user type (i.e. pedestrian crossing reducing pedestrian crashes).

In 2012, Austroads published CRFs for a broad range of road safety engineering treatments (Turner et al., 2012). The effectiveness of treatments was determined through a literature review and synthesis of crash reduction evidence, not through new field experiments. The study considered previous Austroads studies and international literature. Focus was given to studies published since 1999, prioritizing Australia and New Zealand, but also included key overseas data. Not all studies considered corrected for regression-to-mean, confounding, or exposure changes. Similarly, sample sizes were sometimes small, particularly for specific crash types or severities. Some analyses used the “frequency–severity indeterminacy” principle (Hauer 2006, 2009) acknowledging difficulty distinguishing whether frequency or severity truly changed. No new statistical tests or regressions were conducted within this Austroads report; it was limited to synthesizing external quantitative findings.

Both, the NACOE (McHeim, 2020) and Austroads (Turner et al., 2012) reports reviewed in this section are Australian focused including local and international literature. The NACOE report includes some CRFs associated with the Design Exceptions required by TMR. None of the treatments included in the Austroads report match the geometric characteristics related to the DEs required by TMR. Many reports and research papers from overseas were also included without an explanation about how differences in environmental circumstances where considered. For example, a study from Kansas (Zeng et al., 2013) was used to update TMR’s CRFs for shoulders. However, it is not clear how factors that are different between the two jurisdictions, such as speed limits and cross section widths, were considered in the analysis and updates of the CRFs. The NACOE report by McHeim (2020) stated that a workshop and discussions with safety experts were also used to update the CRFs. However, details about how the updates were calculated beyond a weighted average using values from literature were not provided.

### Other relevant studies from Australia

A cross-sectional study using data from Toowoomba developed CMFunctions for roundabouts (Al-Marafi et al., 2020). Several geometric characteristics were found to influence safety along the roundabouts. A similar study from the same group, also using data from Toowoomba, developed CMFunctions for intersections (Al-Marafi et al., 2021). They considered 62 and 44 signalized and un-signalized intersections respectively. CMFunctions were developed for various geometric and operational characteristics by capturing their safety effects. These two studies illustrate the feasibility of developing CMFuntions using local data. They both followed the approach described below in section 3.2. for fully specified models. Unfortunately, the two studies did not consider any of the geometric characteristics from the list of DEs required by TMR and listed in section 2 of this report.

Other studies conducted in Australia involve the analysis of crash frequency for the development of CMFs or CMFunctions including geometric features part of the horizontal alignment, lane characteristics (number of lanes, lane width), vertical alignment, shoulder width, and other context-specific features, such as presence of turns, presence of passing lanes, or intersection characteristics (Tahir et al., 2022a; Tahir et al., 2022b; Howlader et al., 2023; Tahir et al., 2023; Khan et al., 2025).

## Fully specified models

Fully specified models with covariates seek to predict the expected number of crashes as a function of multiple explanatory variables, including traffic exposure, roadway geometric characteristics, situational or environmental factors, and proxies for driver behaviour. These models are commonly developed using count-data methods, such as Poisson or Negative Binomial regression, depending on the relationship between the mean and variance of the response variable. Fully specified crash-frequency models typically adopt a multiplicative functional form, as shown in Equation (2).

Npredicted, x=αAADTβ0×expβ1x1×⋯×expβmxm       (2)

where Npredicted,x is the number of predicted crashes, AADT is the annual average daily traffic, x1,…,xm are covariates associated with road characteristics and other likely contributing factors, β0, …, βm are model parameters or coefficients estimated using field data and methods such the simulated maximum log likelihood. In this functional form, each expβ1x1,…,expβmxm can be viewed as a CMFs, or CMFunction, respectively for each factor from x1 to xm. That is, the structure in Equation (2) exactly matches the relevant conventional approach to use SPFs and CMFs to estimate crashes and the likely effects of countermeasures. Evidence of some of the advantages of using a fully specified model is provided by Lord et al. (2010), who compared this approach to using a baseline model multiplied by CMFs and concluded that the fully specified model produces much less variance.

Although the functional form in Equation (2) is very attractive because in theory it can provide a SPF including CMFunctions, in practice this approach has been shown to be unable to properly capture CMFunctions required for analysis and evaluation of treatment as recommended by the HSM (Chen & Persaud, 2014). Fully specified models often fail in practice because data identification is hard for treatment effects. This is mainly due to selection bias and potential endogeneity, confounding and multicollinearity, little variation across treatment levels, and misspecification of functional forms. Such problems are aggravated when key variables associated with countermeasures or characteristics of interest are eliminated due to statistical insignificance.

## CMFunctions developed through a hierarchical model

While fully specified models theoretically embed CMFs within their parameters, hierarchical models extend this by introducing level-specific variation and enabling local calibration within a unified estimation framework. A hierarchical model is a statistical mechanism that allows multilevel data structures to be properly analysed and estimated. A key characteristic of hierarchical models is how local information is used and analysed. Following the HSM methodology involves using baseline SPFs multiplied by CMFs, as illustrated in Equation (1), where local information is included through the calibration factor, Cx. In contrast, a hierarchical model incorporates local information through sub-hierarchical CMFunctions and by estimating parameters using local factors. Further, the CMFunctions capture continuous correlations involving multiple factors, whereas the Cx is limited to a constant ratio. Hence, a hierarchical modelling approach for the estimation of the effects of road safety countermeasures and treatments is superior to the HSM methodology of baseline SPFs multiplied by CMFs (Chen & Persaud, 2014).

Hierarchical models can restructure variables to safeguard the homogeneity of first-level SPFs. They consider that data heterogeneity is common in road safety analysis thereby hierarchical modelling is an appropriate approach for developing CMFunctions without sacrificing model conformity (Chen & Persaud, 2014).

The development of CMFunctions using a hierarchical approach can be pursued through the following general modelling framework depicted by Equations (3) – (6).

First-Level SPF

The generalised first-level Safety Performance Function (SPF) can be expressed as:

Npredicted=A⋅i=1k(Xi)Bi                                                                                                 (3)

where:

- Npredicted = Predicted outcome (e.g., crash frequency or another dependent variable)

- Xi= AADT variables where i denotes a direction such as a major or minor approach (i=1,2,…,k).

- A = Sub-level CMFunctions (see Equation (4)).

- Bi= Sub-level CMFunctions associated with each Xi (see Equation (5).

- k = The total number of Xi variables included in the model

Sub-Level CMFunctions

Each CMFunctions (A and Bi) is modeled as a function of treatments or roadway characteristics of interest (Zj) or site-specific or contextual variables (Zl) such as type of site (e.g., rural or urban). 
A=α0⋅j=1mexp⁡(αj⋅Zj)                                                                                                 (4)
Bi=β0,i⋅l=1nexp⁡(βl,i⋅Zl)                                                                                                (5)

where α0, αj,β0,i,and βl,i are model coefficients or parameters estimated using data, m is the total number of treatments or roadway characteristics of interest, and n is the total number of site-specific or contextual variables.

Generalised First-Level SPF with Sub-Level Functions

Substituting the sub-level functions into the first-level SPF provides the following equation.

Npredicted=(α0⋅j=1mexp⁡(αj⋅Zj))⋅i=1k(Xi)(β0,i⋅l=1nexp⁡(βl,i⋅Zl))                         (6)
Equation (6) is equivalent to Equation (2) with the added advantage that it is estimated using the hierarchical modelling approach thereby addressing many of the issues discussed above about the development of CMFs and CMFunctions. When the parameters in (6) are estimated, in addition to the p-values and overall goodness-of-fit, the corresponding standard errors must be reported so that the CMFunctions can be used.

## Empirical findings from literature

Table 1 synthesizes some articles related to crash-modelling literature by grouping studies according to their primary topic of interest. This thematic organization highlights common modelling approaches, key explanatory variables, and methodological contributions across major road-safety domains, including geometric design features, operational treatments, and spatial effects. Table 1 also illustrates the dominance of count-data and EB-Ba methods in CMF development, while also identifying emerging approaches that emphasize nonlinear relationships and spatial interactions relevant to CMFunction estimation and DE assessments.

Table 1. Summary of recent empirical studies developing or applying SPFs, CMFs, and CMFunctions.

| Study focus | Unit of analysis | Methods employed | Key variables | Key contributions | Author |
| --- | --- | --- | --- | --- | --- |
| Pavement friction | Highway/road segments | Negative Binomial regression | Traffic composition and volume, segment length, geometric design features, weather conditions | Support with friction-based maintenance and provide practical ways for pavement monitoring | (Subedi, et al., 2025; Cafiso, et al., 2021) |
| Intersections | Signalized, unsignalized, and stop-controlled intersections | Negative Binomial Regression; EB-BA | Traffic exposure, geometric design features, intersection angle | Local crash prediction models and CMFs, along with refining HSM skew-angle guidance. | (Kumfer, et al., 2019; Al-Marafi, et al., 2021) |
| Mid-section U-turns | Roadway segments | Binary logit models and SPFs, CMFs development HSM-consistent | Presence of median U-turn intersection, geometric features, and traffic volume | Provide HSM-compatible CMFs distinguishing safety effects by roadway cross-sections | (Khavarian, et al., 2025; Kay, et al., 2022) |
| Roundabouts | Urban roundabouts | Count models (Poisson, Negative Binomial, Zero-Inflated Negative Binomial) | Traffic volume and composition, roundabout geometric features (diameter, angles, number of lanes) | Development of CMFs for severity specific crashes. Support evidence-based roundabout design guidelines for practitioners | (Alhadidi, et al., 2025; Vinayaraj, et al., 2023; Al-Marafi, et al., 2020; Anjana, et al., 2015) |
| Urban street lighting | Urban arterial road segments | Negative Binomial and Zero-Inflated Negative Binomial regressions | Nighttime crash frequency, presence of standard lighting, average illumination intensity, traffic volume, geometric design features | Framework for prioritizing lighting improvements on urban roads | (Saljoqi, et al., 2021) |
| Paved shoulder width | Rural roadway segments | Descriptive crash pattern analysis and case-control approach | Paved shoulder width and crash occurrence | Identification of nonlinear safety relationship between shoulder width and crash likelihood | (Getnet, et al., 2024) |
| Spatial interactions for CMFs development | Roadway segments | Spatially adaptative EB, Spatial Random-Effects Zero-Inflated count models | Traffic composition and volume, geometric features of road segments, operational features | Demonstration of the necessity of spatial interactions in modelling framework for CMFs and SPFs development | (Almasi, S. A., & Yang, J. 2026; Galgamuwa, et al., 2021; Noland, R. B., & Adediji, Y. 2018) |

# Development and implementation roadmap

The following aspects should be considered in the development of CMFS and CMFunctions, as well as in the estimation of crash-frequency and severity models. While analysing a larger number of road segments or clusters may increase the overall size of the dataset, further stratification by road class, crash type, severity level, vehicle type, or user group reduces the effective sample size available for estimation within each subgroup. Given the substantial heterogeneity present across road networks, traffic conditions, crash mechanisms, and user populations, careful modelling and analytical decisions are required to balance statistical reliability with behavioural realism.

## Network definition and scope

The first step is to define what population of road elements the SPFs represent, what safety process is being modelled, and how results will be used. This step ensures internal validity, transferability, and policy relevance. SPFs are not generic predictions tools but purpose-built, so it must be explicitly stated the main function of the SPF that is being developed. If the SPF is being used as a baseline safety prediction it implies a strong emphasis on reference conditions and minimal embedded covariates. Another option is to use the SPF as a countermeasure evaluation, which implies that variables of interest must be separable from the base SPF and segmentation must not absorb treatment variations. Also, SPFs can be used for policy or Design Exceptions (DEs) assessment, meaning that CMFunctions are preferred over segmentation with clear valid ranges required.

Another step required during the first steps of the analysis is to define the statistical population from which segments are drawn. This procedure involves defining: (i) the facility type (e.g., rural or urban roads, freeway segments, ramps, divided or undivided), this produces interpretable CMFs or CMFunctions, (ii) the traffic control and operational regime (e.g., signalized or not, access-controlled or non-access controlled) controlling the crash generation mechanisms, (iii) the area or context (e.g., urban or suburban, functional class-specific) which controls for driving behaviour and exposure to related features, and (iv) the data source (e.g., crash reporting system, traffic data source, design inventory source) which defines transferability boundaries and yields calibration logic.

Along with previous steps, it is required to define the temporal scope. This procedure is not trivial, as it must verify the consistency of major geometric changes, policy shifts in terms of operational features, and stability of traffic patterns.

Finally, the crash types and severity scope must be defined. Crash inclusion rules must be set, which answers what type of crashes are considered in the analysis (e.g., injury crashes, property damage, fatal crashes, specific types). This is mainly due as different crash types respond differently to design features and CMFs are crash-type specific.

## Segmentation analyses

Table 2 presents a functional classification of the road network into road stereotypes that are typically considered in Australia for separated crash analysis. These stereotypes are intended to capture broad differences in traffic function, access control, speed environment, and geometric design, providing a basis for stratified safety analysis and the development of appropriate SPFs, CMFs, and CMFunctions. By grouping segments with similar operational and design characteristics, the classification seeks to reduce within-group heterogeneity while maintaining sufficient variability for statistical estimation (Austroads 2016; Austroads 2021).

Table 2. Functional roadway stereotypes used to define homogeneous facility types for SPF development

| Stereotype | Typical Context | Key Geometric / Functional Traits |
| --- | --- | --- |
| - Urban arterial | Within built-up area | 4+ lanes, 60–80 km/h, signalised intersections, curb-and-channel, high access density |
| - Urban collector | Suburban / fringe | 2 lanes, 50–60 km/h, mixed land use, moderate access, some signals/roundabouts |
| - Urban local | Residential streets | 1–2 lanes, 40–50 km/h, high driveway density, traffic calming features |
| - Rural highway | Interurban routes | 2–4 lanes, 80–110 km/h, sealed shoulders, long sight distance, limited access |
| - Rural arterial | Connects regional centres | 2 lanes, 80–100 km/h, narrow shoulders, moderate curves |
| - Rural collector / local | Access to farms or small towns | 1–2 lanes, 60–100 km/h, unsealed or partially sealed, low AADT |
| - Freeway / motorway | Controlled access corridors | 4+ lanes, 100–110 km/h, grade-separated, full access control, median separation |
| - Rural remote | Outback and low-volume roads | Unsealed or lightly sealed, >100 km/h, limited delineation, very low traffic volumes |
| - Urban motorway corridor | Inner-city tollways or grade-separated arterials | 80–100 km/h, limited access, complex interchange systems |

In this sense, Balusu et al. (2017) conducted an analysis of speed profiles for accident occurrence modelling considering that road segments are not fixed by geometry alone. Instead, the segments were created using the free-flow speed profile as a proxy for homogeneity in hilly terrain. The authors claimed that geometry changes so often that classic homogeneity-based segmentation (e.g., by length) creates many tiny, biased sections.

## Model specification and SPF estimation

This step represents the analytical core of the development roadmap. First, the dependent variable and the exposure structures are defined. The response is typically crash frequency, while exposure is represented through traffic volumes, segment length, or intersection counts. Decisions during this step include whether to model total crashes or disaggregate them by severity or crash types, along with how to normalize exposure so predictions are interpretable and scalable.

The statistical framework and functional forms depend mainly on the behaviour of the response variable. The most common specifications include Poisson or Negative Binomial models, with this latter preferred when overdispersion is present. Also, the log-linear form is standard, which allows for multiplicative effects and direct compatibility with CMFs and CMFunctions. This stage involves the decision whether to use fixed-effects, random-effects, or hierarchical formulations depending on the treatment to the unobserved heterogeneity across segments, corridors, or regions.

The specification of covariates usually includes traffic volume and segment length for exposure, along with design elements (e.g., lane width, shoulder width, curvatures, grades) and operational features. Some variables that are directly related to countermeasures should be handled carefully. They should be excluded from the base SPF or included in a constrained form, to avoid confounding and to preserve the interpretability of later CMFs developed.

It is worth mentioning that some researchers have applied crash frequency analysis by using advanced statistical frameworks that allow for multiple response, which allows for the evaluation of safety measures considering a multivariate framework with different severity crashes. For instance, Zhang et al. (2022) analysed the differences in crash frequency addressing for daytime versus nighttime with multiple responses considering light crashes, minor crashes, severe crashes, and very severe crashes in a multivariate framework. Similarly, Wang et al. (2020) analysed contributing factors on crash frequency by crash types (e.g., rear-end crashes, bumping-guardrail crashes, non-casualty crashes, and casualty crashes). Both cases demonstrate the use of advanced multivariate tools that tackle issues with traditional models with single response.

## Calibration and transferability assessments

This step ensures that SPFs accurately reflect the crash experience of the target segment or roadway network and can be used in practice. Even well-specified SPFs may systematically over-predict or under-predict crashes when applied to new contexts. Hence, this step compares observed crashes to SPF-predicted crashes across representative samples to determine whether systematic bias exists. Calibration of SPFs is typically implemented using a multiplicative factor which is defined as the ratio of observed to predicted crashes. This factor depends on data availability and heterogeneity. The main objective is to preserve the structure of the SPF while aligning its scale with local conditions.

## Sample size and number of sites

The HSM states that SPFs should be developed using enough sites that are homogeneous. This requirement is framed primarily in terms of stability and bias of the tools developed, considering generalizability and inference properties. For local SPF development, the HSM states that analysis should be conducted with enough sites ensuring reliability. For SPF calibration some thresholds are given (e.g., minimum of 30 sites for calibration, and more than 50 or 100 sites preferred for stable calibration factors). The samples should be representative of the facility type, traffic range, and geographic context, which are defined at the beginning of the analysis.

Some guidelines appear that refer mostly to statistical methods rather than HSM guidelines. For instance, when models include multiple covariates, analysts often prefer at least 10 or 20 sites per estimated parameter to avoid overfitting and unstable coefficients (Ogundimu et al., 2016). Furthermore, this issue is commonly acknowledged as a limitation in studies conducted in literature. For instance, Alhadidi et al. (2025) denotes that the limited sample size of an analysis conducted for crash severity at urban roundabouts may not provide high robustness of the estimates. Similarly, Al-Ahmadi et al. (2021) state that increasing sample size could result in better calibrations and reduce overestimation issues and benefit transferability to other contexts without requiring complex recalibration methods.

# Illustrative estimation of SPF and CMFs

According to HSM guidelines, the roadmap for SPF development involves the following steps: (i) Definition of purpose, including clarification of site type and data scope (e.g., highway facility type, crash types, and analysis period). This step also entails confirming homogeneity across sites and documenting screening rules, such as procedure for handling missing data and other inconsistencies. (ii) Selection of exposure measures and the SPF functional form requires careful examination of the dependent variable, as its structure and statistical properties determine which modelling frameworks are most appropriate for explaining crash frequency. (iii) Estimation of the baseline model according to the assumed distribution of the dependent variable, followed by assessment of model adequacy through diagnostics addressing overdispersion, multicollinearity, and residual behaviour. (iv) Identification of CMF-eligible variables, focusing on modifiable factors. At this stage, covariates are classified into treatment variables, representing geometric or operational characteristics subject to design or policy interventions, and contextual variables, which serve to control confounding effects. To demonstrate the variation in results and insights produced by alternative modelling and analysis methods, the following comparison examines fixed-effects and random-parameters models with respect to their goodness-of-fit and parameter estimates (Ahern, et al., 2025).

## Washington dataset

The Washington dataset comprises crash frequency observations for 275 homogeneous road segments, together with a set of geometric and operational characteristics. The variables include segment length (in miles), average annual daily traffic (AADT), number of lanes, lane width, posted speed limit (miles per hour), number of interchanges, number of curves, average median shoulder width (in feets), tangent length (in miles), maximum grade difference in the road segment (in %), and the type of slope (flat or slight), along with the daily percentage of trucks, and average levels of snowfall and precipitation across segments. .

Figure 1 presents the histogram of crash frequency across road segments. Only 6.2% of the segments record zero crashes, indicating a relatively low prevalence of zero outcomes. Consequently, zero-inflated count models are not employed, as their validity hinges on the presence of significant zero inflation and on compelling evidence for a bifurcated data-generating process, neither of which is observed in this datasetn contrast, prior studies have employed zero‑inflated count models in road‑safety analyses involving specific crash types that generate substantially higher proportions of zero counts across segments, which may improve statistical fit but nevertheless introduce an ill‑defined modelling paradigm when the zero‑inflation component lacks a defensible theoretical basis (Champahom, et al., 2023; Saljoqi, et al., 2021; Galgamuwa, et al., 2021).

A hierarchical model is fitted, accounting for the multilevel structure described in Equation (6). Table 3 presents the parameter estimates of the hierarchical approach, along with comparative results from a model suggested in the literature, specifically the one proposed by Simon Washington et al. (2020). The hierarchical approach yields a higher BIC and log-likelihood; however, the difference is marginal, particularly given the inclusion of random parameters and variables at both levels of the model specification.

The specification obtained from the hierarchical approach reveals several statistically significant geometric design variables. Following Equations (4)-(6), the resulting specification is presented in Equation (7), incorporating a two-level structure. It is important to note that this specification is consistent with the guidelines of the HSM, particularly regarding the multiplicative structure of CMFunctions or CMFs and the base SPF.

Np=exp-10.38+0.42X1+0.09X2-0.05X3+0.08X4-0.44X5+0.10X6×(AADTi)5.39×exp-0.01X7            (7)

Following the guidelines of the HSM for CMF development, the expression in Equation (7) can be applied analogously to Equation (1), incorporating both the SPF and the CMFs derived from the hierarchical structure. In this context, each coefficient in the lower-level expressions can be used to estimate CMFs or CMFunctions based on their corresponding parameter estimates. Equation (8) presents the formulation used to estimate the effect of a variable changing from value a to b, holding all other factors constant.

CMFa→b=NbNa=exp⁡(a1b)exp⁡(a1a)=expa1b-a   (8)

The estimation results indicate that the number of interchanges has a coefficient of α=0.42 , implying that a one-unit increase in the number of interchanges is associated with an approximate 52% increase in crashes. Similarly, a one-unit increase in the number of curves in a road segment leads to an increase of about 9.4% in crashes (α=0.09). In contrast, increasing lane width by one foot is associated with a reduction of approximately 4.8% in crashes (α=-0.05). An increase of 1% in the maximum grade difference within a road segment is associated with an 8.3% increase in crashes (α=0.08). Additionally, a one-foot increase in shoulder width corresponds to a reduction of about 35% in crashes (α=-0.44). Furthermore, it is important to note that, in Equation (7), the terms associated with AADT function as the base SPF, consistent with the recommendations of the HSM outlined in Equation (1).

A key advantage of the hierarchical approach is its ability to mitigate omitted variable bias. Depending on the data structure, failing to account for heterogeneity may lead to coefficients associated with local factors absorbing the effects of contextual variables, thereby producing biased estimates or resulting in over- or underestimation of parameters (AASHTO, 2010; Lord, 2010). This issue is illustrated by the estimated effect of the number of curves across models.

The specification derived from the literature suggests that an increase in the number of curves reduces crash frequency, which is counterintuitive and contrasts with established findings in the geometric design literature. In contrast, the hierarchical approach indicates that an increase in the number of curves in road segments leads to higher crash frequencies, a result that is consistent with prior studies in this geometric feature for road design (Wu et al., 2017; Gooch et al., 2016).

Figure 1: Distribution of crashes per road segment in the Washington dataset.

Table 3: Specification found for the hierarchical framework compared to literature. The marks * denote significance at 1% (***), 5% (**), and 10% (*).

|  | Specification by Washington et al. |  |  |  |  | Specification provided by hierarchical framework |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Coefficient | p-value |  | Variable |  |  | Coefficient |  | p-value |  |  |
| Constant (α0) | 2.81 | <0.001*** |  | Constant (α0) |  |  | -10.38 |  | <0.001*** |  |  |
| Interchanges | 0.15 | 0.01** |  | Interchanges (X1) |  |  | 0.42 |  | <0.001*** |  |  |
| Curves | -0.16 | <0.001*** |  | Curves (X2) |  |  | 0.09 |  | <0.001*** |  |  |
| 1 if segment is considered as low precipitation, 0 otherwise | -0.41 | <0.001*** |  | Width per lane (X3) |  |  | -0.05 |  | 0.02** |  |  |
| Number of grade breaks in the segment | -0.05 | 0.02** |  | Maximum grade difference in the segment X4 |  |  | 0.08 |  | 0.09* |  |  |
| Friction | -0.01 | 0.05* |  | Shoulder width (X5) |  |  | -0.44 |  | 0.04** |  |  |
| Expose | 2.70 | <0.001*** |  | Tangent length X6 |  |  | 0.10 |  | 0.06* |  |  |
| 1 if the segment is considered as high snow, 0 otherwise | -0.18 | 0.01** |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Constant (β0) |  |  | 5.39 |  | <0.001*** |  |  |
|  |  |  |  | 1 if the slope in the segment is flat, 0 otherwise (X7) |  |  | -0.01 |  | 0.06* |  |  |
| Std. Deviation of random parameters |  |  |  |  |  |  |  |  |  |  |  |
| Std. Dev. (first level) |  |  |  |  |  |  | 0.55 |  | <0.001*** |  |  |
| Std. Dev. (sub level) |  |  |  |  |  |  | 0.24 |  | <0.001*** |  |  |
| Goodness-of-fit |  |  |  |  |  |  |  |  |  |  |  |
| BIC | 1993 |  |  |  |  | 2034 |  |  |  |  |  |
| Log-Likelihood | -956 |  |  |  |  | -986 |  |  |  |  |  |

Offset logAADT×Segment Length

Npredicted=Offset×exp{-7.977+0.098X1+0.178X2+0.036X3+0.118X4-0.010X5}   (7)

Npredicted=Offset×exp0.069±0.073X1×exp-8.037+0.181X2+0.043X3+0.135X4-0.008X5     (8)

exp⁡{0.0069±0.073X1}X1X1

# Discussions

The above sections examined a range of quantitative methods available for estimating the safety effects of roadway geometric characteristics, with particular emphasis on the suitability for supporting DE assessment and evidence-based decision-making. Rather than advancing new predictive models or definitive CMFs, the analysis was intended to clarify what current methods can realistically deliver, and how they align with guidance provided by the HSM and related frameworks.

A central insight emerging from the reviewed literature is that methodological sophistication does not automatically translate into practical usefulness for DE assessments. While recent studies increasingly apply random-parameter, hierarchical, spatial, or fully Bayesian models, their successful implementation depends on data quality and completeness, sample size, and sufficient variation in the geometric characteristics of interest. In many real-word applications, particularly those involving isolated or infrequently occurring DE, these requirements are not always met. As a result, simpler modelling frameworks, such as fixed effects count models, remain prevalent not because they are theoretically optimal, but due to their robustness, transparency, and interpretability under realistic data conditions. See for instance (Almasi, S. A., & Yang, J. 2026; Khavarian, et al., 2025) in which the authors highlight issues with overfitting and limited sample sizes for specific configurations of crashes that yield issues with generalizability of results.

The review shows that CMFs and CMFunctions remain the predominant tools for translating statistical models into actionable safety insights for decision-makers. However, it also reinforces long-standing concerns regarding their interpretation and application. Many geometric characteristics relevant to DEs are highly correlated with broader design standards and operational context. Cross-sectional models often struggle to isolate their marginal effects with sufficient precision, resulting in large uncertainty around estimated CMFs. As a result, issues with transferability, generalization, and bias in inference are acknowledged in literature denoting limitations derived from data related issues (e.g., availability, quality, completeness, retrieving procedures) (Alhadidi, et al., 2025; Al-Marafi, et al., 2024).

## Alignment with HSM guidelines and current practice

The findings of this review are broadly consistent with the philosophy underpinning the HSM, which emphasizes parsimony, transparency, and compatibility with EB procedures. These favours modelling approaches that can be readily interpreted and calibrated for decision-making units. Although more advanced methods are acknowledged in the research literature, they are not embedded in routine practice, largely due to implementation challenges and data requirements.

From this perspective, this review suggests that current practice is not lagging methodological advances but rather reflects a pragmatic balance between analytical rigor (in statistical terms) and operational feasibility for decision-makers. The continued reliance on NB-based frameworks and multiplicative CMFs is therefore best viewed as a rational response to data limitations, rather than as resistance to innovation.

The comparison between traditional count models and hierarchical formulations highlights an important distinction between statistical fitness and inferential usefulness. Although not mentioned explicitly in the HSM as guidelines, hierarchical models offer a principled mechanism for pooling information across contexts, allowing parameter estimates to borrow strength across groups while preserving local variation, along with offer suitable model specification and settings that might be time consuming through traditional methods (Ahern, et al., 2025). This property is especially relevant when geometric features exhibit limited variability or when some facility types are sparsely represented. In such cases, hierarchical CMFunctions can stabilize estimates and reduce sensitivity to arbitrary segmentation choices, addressing a recurring limitation of cross-sectional CMF development noted in literature.

Furthermore, hierarchical models align closely with the guidelines of the HSM, which recommend estimating geometric design effects alongside a calibration factor to account for contextual characteristics in the development of CMFs or CMFunctions. This framework is naturally accommodated by the multiplicative specification of the hierarchical approach, enabling the simultaneous estimation of crash modification and calibration factors within a single-stage approach.

## Implications for future research and policy development

The reviewed evidence indicates that future progress in DE-related safety analysis is likely to depend more on strategic data development and study design than on incremental changes in statistical methodology. Longitudinal datasets, consistent recording of geometric features, and targeted BA evaluations would substantially enhance the ability to estimate reliable CMFs for DE-relevant characteristics. Authors have attributed this limitation primarily to the reliance on approximations or too costly data-collection procedures, which may bias results and require substantial time and computational effort, thereby limiting their practical applicability for countermeasure design (Cafiso, et al., 2021; Khavarian, K. & Sahebi, S., 2025; Fitzpatrick, et al., 2021). Hierarchical and Bayesian approaches remain promising, particularly when data can be pooled across regions or over time, but their use should be motivated by clear analytical gains rather than methodological novelty.

For agencies such as TMR, the implications are twofold. First, cross-sectional modelling and CMFs can provide valuable screening-level insights into relative safety sensitivity, supporting prioritization and qualitative risk assessment for DE evaluations. Second, when DE decisions carry high safety or economic stakes, these tools should be complemented by more targeted evaluation methods, engineering judgement, and uncertainty-aware decision frameworks.

In summary, this scoping review underscores that the principal challenge in applying quantitative safety models to DE assessment lies not in the absence of sophisticated methods, but in matching analytical tools to realistic data environments and decision contexts. Existing modelling approaches, when applied carefully and interpreted conservatively, can offer meaningful insights into the relative influence of geometric design features. However, their outputs should be viewed as part of a broader evidence base, rather than as standalone predictors of safety performance. This perspective is consistent with both the HSM framework and the broader body of international road safety research.

# Recommendations

The above sections provide a brief overview of the various methods available in literature and commonly used in practice for the development of CMFs and CMFunctions. This overview includes details about the characteristics and challenges involving various methods. Despite the vast number of CMFs provided by the HSM and FHWA’s Clearinghouse as well as the statistically sound methods that are recommended to implement them, there is sufficient evidence including both theoretical and empirical analyses that advise the development of CMFunctions using local data and hierarchical methods. This report also provides an overview of the hierarchical methods including a general formulation that can be used for the corresponding development of local CMFunctions. Given the limitations of existing CMFs, it is recommended to prioritize developing local CMFunctions using hierarchical models to improve predictive accuracy and resource allocation, along with addressing potential heterogeneity of the behaviour given specific context-conditions in road safety analysis.

A summary of the reviewed methods and approaches, including the key advantages and disadvantages of each, is presented in Table 4. Overall, no single methodological approach is universally optimal for the estimation of safety effects of roadway geometric characteristics. Simpler approaches offer transparency and have lower data requirements, but they are highly vulnerable to bias and unobserved changes. Other methods address many limitations and remain robust and operationally feasible under current practice, particularly when aligned with HSM guidance. Cross-sectional and fully specified models enable the evaluation of a broader set of design characteristics and marginal effects, but they lack causal interpretation and require specific data availability conditions. More advanced methods, such as hierarchical frameworks, provide a principled mechanism to account for heterogeneity, uncertainty, and contextual dependence, offering substantial analytical advantages conditional on data structures. Consequently, the appropriate choice of the method depends on the decision context, data quality, and risk associated with the DE under consideration. This latest reinforces the need for a pragmatic application of safety modelling rather than reliance on a single analytical approach.

Table 4: Summary of methods used for estimating CMFs and CMFunctions.

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Observational Before-After | - Controls for time-invariant unobserved characteristics - Intuitive and policy friendly - Easy to communicate - Directly measures observed changes following an intervention | - Vulnerable to regression-to-the-mean - Confounded by traffic growth, land-use changes, regulation shifts, vehicle technology trends - Infeasible for geometric features that cannot be altered independently - Limited availability to control for changes in exposure |
| Comparison group | - Controls for broad temporal trends affecting safety - More robust than BA when a well-matched comparison group exists - Direct extension of BA logic | - Assumes that treated and comparison sites evolve identically over time - Sensitive to poor selection of comparison sites yielding biased results if not well-matched - Weak control of regression-to-the-mean bias |
| Yoked comparison | - Lower data requirements than comparison-group designs; conceptually simple - Practica when data availability is limited | - Highly sensitive to matching quality - Biased estimates if the paired site is not truly comparable |
| Naïve approach | - Simple to apply with minimal data requirements - Useful as descriptive or illustrative baseline | - Severely biased due to regression-to-the-mean - Assumes a stable crash-generation process - Tends to overestimate treatment effectiveness |
| Empirical Bayes | - Explicitly corrects for regression-to-the-mean bias - Combines observed crash frequencies with SPF-based expectations - Accounts for site characteristics - More reliable than Naïve approach, comparison group and Yoked comparison | - Requires well-specified SPFs from reference sites - Relies on model assumptions external to treated sites |
| Full Bayes | - Jointly estimates model parameters and site-specific crash frequencies - More statistically coherent than EB - Flexible framework for heterogeneity and complex structures | - Computationally intensive - Higher technical expertise required - Greater data availability requirements |
| Cross-sectional studies | - Cross-effective and scalable - Can analyse many sites simultaneously - Enables estimation of marginal effects of geometric features - High statistical power with large datasets | - Vulnerable to omitted variables - Associational results and not causal interpretations - Sensitive to data availability |
| Fully specified models | - Directly estimation of multiple covariates simultaneously - Embed CMFs and CMFunctions within model coefficients | - Identification problems for treatment effects - Sensitive to endogeneity, multicollinearity, and selection bias - Key variables may be dropped due to statistical insignificance |
| Hierarchical models | - Explicitly accounts for multilevel data structures and heterogeneity - Local calibration within one framework - More flexible and realistic than fixed-effect models - CMFunctions vary with context instead of being constant | - More complex to specify and estimate - Higher data and computational requirements - Benefits depend on sufficient variation and sample size |

# Proposed Scope of Work

Considering the above material, the following tasks are recommended for a future research project. Research findings and results throughout the project will shape subsequent tasks and outcomes.

Task 1: Data preparation and assessment

TMR will provide the data required for this project, including historical crash records, traffic volumes, and corresponding roadway geometric characteristics. TMR can supply the raw source datasets, and QUT will undertake the necessary data integration and preparation. QUT will also conduct a detailed assessment of the available data to inform subsequent tasks, noting that the scope of these tasks will depend on the completeness and characteristics of the datasets provided.

QUT expects to receive recent roadway characteristic data, including geometric attributes; historical crash data, including narrative descriptions where required to verify crash coding; and traffic volume information. To enable effective data integration, the datasets must include consistent identifiers such as spatial location information for road segments, crash locations, and volume sites. Similarly, temporal attributes are required, such as the year or date of each crash and the associated traffic volume period.

Tasks 2 - Development of Safety Performance Functions (SPFs) and Crash Modification Factors/Functions (CMFs):

A wide range of variables—including road stereotypes, crash types, traffic volumes, vehicle classifications, and geometric attributes—may be used to develop relevant safety-performance models. However, the availability, completeness, and quality of the dataset will ultimately constrain the number and type of models that can be reliably estimated. Therefore, the tasks outlined below represent an aspirational scope that will be refined based on data availability and preliminary modelling results. The number of CMFs developed, and the specific geometric characteristics incorporated will depend on the size and richness of the dataset.

In close collaboration with TMR, QUT will investigate the following sets of SPFs (Item 2A–2D below) and their corresponding CMFs. Each SPF will aim to include, where possible, the geometric characteristics most associated with the need for Design Departures. Ideally, the dataset should include as many of these characteristics as possible; however, not all characteristics are required to develop valid SPFs and CMFs. Many geometric elements may not be statistically significant for certain crash types or road stereotypes and can therefore be excluded.

Over time, as more data and information become available, additional models can be developed to include new information and variables and/or the estimated models can be updated to seek to include additional characteristics when relevant.

Design elements within scope of the project:

1. Lane width, shoulder width, median width

2. Intersection type and turn configuration

3. Horizontal curve radius

4. Superelevation / adverse crossfall

5. Roadside treatment types

6. Stopping Sight Distance (SSD)

Other design elements:

7. Vertical curve radius

8. Longitudinal Road Grade

9. intersection sight distances (SISD, MGSD)

10. Aquaplaning risk

The final selection of geometric elements to be modelled will be confirmed during the initial phase of the project following a detailed investigation of the available data and preliminary statistical analysis. This analysis will identify which roadway characteristics are likely to influence crash occurrence and severity for different road stereotypes and crash types. It is anticipated that certain crash types may not be affected by geometric attributes; in these cases, the statistical modelling will reveal which factors meaningfully contribute to safety performance.

Item 2A – SPFs and CMFs considering road Stereotype

SPFs and CMFs will be estimated for each of the following TMR controlled road stereotypes.

1.	Rural roads/highways

2.      Intersections such as Roundabouts, T-intersections, Four-leg intersections on rural arterial roads/highways

Item 2B - SPFs and CMFs considering crash type

SPFs and CMFs will be estimated for applicable crash type groups for each road stereotype where relevant. For example, head-on collisions on motorways may not need to be considered because they are unlikely to occur on motorways, unless wrong way driving. Statistical analysis will be used to determine which crash types are relevant for a particular model.

Item 2C - SPFs and CMFs considering road stereotype and crash type and vehicle type

SPFs and CMFs will be estimated for all vehicle types, light vehicles as well as heavy vehicles groups if statistically significant.

a) Light vehicles such as:

- Passenger vehicle (5.2 m)

- Passenger vehicle towing trailer (17.6 m)

b) Small heavy vehicles and other large heavy vehicles such as:

- Service vehicle (8.8 m)

- Single unit truck/bus (12.5 m)

- Long rigid bus (14.5 m)

- Articulated bus (19 m)

- Prime mover and semi-trailer (19 m)

- Prime mover and long semi-trailer (25 m)

- B-double (25 m)

- B-double (26 m)

- A-double (Type I road train) (36.2 m)

- B-triple (35.4 m)

- A-triple (Type II road train) (53.4 m)

Item 2D - SPFs and CMFs considering severity level

SPFs and CMFs will be estimated for each group of the following crash severity group types.

a) Fatal and serious injury crashes

- Fatal

- hospitalization

b) All crash severity types

- Fatal

- hospitalization

- Minor injuries

- Medically treated

Task 3- Final Report and presentation

QUT will deliver a written technical report including project fundings and details about how all the above tasks were completed with results, models, statistics, and software tools to enable the analysis of data to reproduce results and repeat the analyses in the future with updated and/or additional data. Similarly, QUT will deliver a final presentation summarising the project results and findings.

Budget

The following table provides a list of key milestones, deliverables, and the corresponding budget request.

| Milestone/ Deliverable | Expected Completion date | Budget Breakdown |
| --- | --- | --- |
| Data preparation and assessment | 30/11/2026 | $ 60,000 |
| Model development (SPFs and CMFs) ) for first set of outcomes - key design elements: 1. Lane width, shoulder width, median width 2. Intersection type and turn configuration SPFs and CMFs considering crash type SPFs and CMFs considering road stereotype and crash type and vehicle type SPFs and CMFs considering severity level | 30/03/2027 | $ 100,000 |
| A detailed technical report of project and its finding | 30/05/2027 | $ 15,000 |
| TMR review of the technical report (hold point) | 30/06/2027 | $ 0.00 |
| Data preparation and assessment | 30/11/2027 | $60,000 |
| Model development (SPFs and CMFs) ) for second set of outcomes - key design elements 3. Horizontal curve radius 4. Superelevation / adverse crossfall SPFs and CMFs considering crash type SPFs and CMFs considering road stereotype and crash type and vehicle type SPFs and CMFs considering severity level | 28/04/2028 | $ 100,000 |
| A detailed technical report of project and its finding | 30/05/2028 | $ 15,000 |
| TMR review of the technical report (hold point) | 30/06/2028 | $ 0.00 |
| Data preparation and assessment | 30/11/2028 | $60,000 |
| SPFs and CMFs considering road Stereotype 5. Roadside treatment types 6. Stopping Sight Distance (SSD) SPFs and CMFs considering crash type SPFs and CMFs considering road stereotype and crash type and vehicle type SPFs and CMFs considering severity level | 30/04/2029 | $ 100,000 |
| A detailed draft technical report of project and its findings | 30/05/2029 | $ 15,000 |
| Updated final technical report and presentation | 30/06/2029 | $ 0.00 |

# References

Al-Marafi, M. N., Somasundaraswaran, K., & Ayers, R. (2020). Developing crash modification factors for roundabouts using a cross-sectional method. Journal of traffic and transportation engineering (English edition), 7(3), 362-374.

Al-Marafi, M. N., Somasundaraswaran, K., & Bullen, F. (2021). Development of crash modification factors for intersections in Toowoomba city. International Journal of Urban Sciences, 25(1), 104-123.

American Association of State Highway and Transportation Officials. (2010). Highway safety manual (1st ed.). Washington, DC: Author.

Austroads. (2023). Guide to Road Design. https://austroads.gov.au/publications/road-design/agrd-set

Chen, Y., and B. Persaud (2014). Methodology to develop crash modification functions for road safety treatments with fully specified and hierarchical models. Accident Analysis & Prevention 70, 131-139.

Chin, H.C., and Huang, H. (2008). Modeling Multilevel Data in Traffic Safety: A Bayesian Hierarchical Approach. Transportation Accident Analysis and Prevention. Nova Science Publishers, Inc, New York, USA, 53–106.

Elvik, R. (2005). Introductory guide to systematic reviews and meta-analysis. Transportation research record 1908(1), 230-235.

Elvik, R. (2009). Developing accident modification functions: exploratory study. Transportation research record 2103(1), 18-24.

Elvik, R. (2015). Methodological guidelines for developing accident modification functions. Accident Analysis & Prevention 80, 26-36.

Elvik, R., A. Hoye, T. Vaa, and M. Sorensen. (2009). Handbook of Road Safety Measures. Emerald Group Publishing, Bingley, United Kingdom.

Elvik, R. (2024). A comparative analysis of accident modification functions for traffic law enforcement [Article]. Accident Analysis and Prevention, 195, Article 107415.

Federal Highway Administration (FHWA) (2011). Crash modification factors clearinghouse. FHWA, U.S. Department of Transportation, Washington, DC, USA, http://www.cmfclearinghouse.org (accessed 01.10.25).

Goldstein, H. (1999). Multilevel Statistical Models. Institute of Education, Multilevel Model Project, London, UK.

Hauer, E. (2006). The frequency-severity indeterminacy [unreported crashes]. Accident Analysis and Prevention, 38(1), 78–83.

Hauer, E. (2009). Speed and safety. Transportation Research Record, 2103, 10–17.

Hosking, J., Mackie, H., Macmillan, A., van der Werf, B., Smith, M., Witten, K., & Woodward, A. (2023). Effects of Te Ara Mua – Future Streets suburban street retrofit on traffic speed and volume: Controlled before-after study. Journal of Transport and Health, 30, 101601.

Harkey, D.L., Srinivasan, R., and Baek, J. (2008). Accident Modification factors for traffic engineering and ITS improvements. National Cooperative Highway Research Program (NCHRP) Report 617. Transportation Research Board of the National Academies, Washington, DC, USA, https://www.trb.org/publications/nchrp/nchrp_rpt_617.pdf (accessed 01.10.25).

Hauer, E., Bonneson, J. A., Council, F., Srinivasan, R., and Zegeer, C. (2012). Crash Modification Factors: Foundational Issues: Foundational Issues. Transportation Research Record: Journal of the Transportation Research Board, 2279(1), 67-74. https://doi.org/10.3141/2279-08 (accessed 01.10.25).

Lord, D., Persaud, B., Washington, S.P., Ivan, J.N., Lyon, C., and Jonsson, T. (2008). Methodology to predict the safety performance of rural multilane highways. NCHRP Web-only document 126, 53–59, http://onlinepubs.trb.org/onlinepubs/nchrp/nchrp w126.pdf (accessed 01.10.25).

McCoy, E., & Graham, D. J. (2025). Evaluation of the causal impact of recreational marijuana legalisation on traffic safety in the US. Accident Analysis and Prevention, 220, 108106.

McHeim, B. (2020). R87: Development of Crash Reduction Factors. National Asset Centre of Excellence. ARRB Project No.: 014905.

Sacchi, E., and T. Sayed (2014). Accounting for heterogeneity among treatment sites and time trends in developing crash modification functions. Accident Analysis & Prevention 72, 116-126.

Sayed, T., and de Leur, P. (2008). Collision modification factors for British Columbia. British Columbia Ministry of Transportation & Infrastructure, Victoria, British Columbia, Canada, https://www2.gov.bc.ca/assets/gov/driving-and-transportation/transportation-infrastructure/engineering-standards-and-guidelines/traffic-engineering-and-safety/highway-safety/cmfs_for_bc_2008.pdf (accessed 01.10.25).

Sawalha, Z., and T. Sayed (2001). Evaluating safety of urban arterial roadways. Journal of Transportation Engineering 127(2), 151-158.

Turner, B., Steinmetz, L., Lim, A., and Walsh, K. (2012). Effectiveness of Road Safety Engineering Treatments. Austroads. Publication No. AP-R422-12. ISBN 978-1-921991-60-8.

Wang, J.-H., Abdel-Aty, M., and Lee, J. (2016). Examination of the Transferability of Safety Performance Functions for Developing Crash Modification Factors: Using the Empirical Bayes Method. Transportation Research Record: Journal of the Transportation Research Board, 2583(1), 73-80. https://doi.org/10.3141/2583-10 (accessed 01.10.25).

Wu, L., Lord, D., and Geedipally, S. R. (2017). Developing Crash Modification Factors for Horizontal Curves on Rural Two-Lane Undivided Highways Using a Cross-Sectional Study. Transportation Research Record: Journal of the Transportation Research Board, 2636(1), 53-61. https://doi.org/10.3141/2636-07 (accessed 01.10.25).

Zeng, H, Schrock, S & Mulinazzi, T. (2013). Evaluation of safety effectiveness of composite shoulders, wide unpaved shoulders, and wide paved shoulders in Kansas, K-TRAN: KU-11-1, Kansas Department of Transportation, Topeka, KS, USA.

Zegeer, C.V., J.R. Stewart, F.M. Council., and D.W. Reinfurt. (1991). Safety Effects of Geometric Improvements on Horizontal Curves. Report UNCHSRC-91. University of North Carolina, Chapel Hill.

Zhu, X., McCoy, E., & Graham, D. J. (2025). Safe streets for cyclists? Quantifying the causal impact of cycling infrastructure interventions on safety. Accident Analysis and Prevention, 220, 108168.

Tahir, H. B., Haque, M. M., Yasmin, S., & King, M. (2022a). A simulation-based empirical bayes approach: Incorporating unobserved heterogeneity in the before-after evaluation of engineering treatments. Accident Analysis & Prevention, 165, 106527.

Tahir, H. B., Washington, S., Yasmin, S., King, M., & Haque, M. M. (2022b). Influence of segmentation approaches on the before-after evaluation of engineering treatments: A hypothetical treatment approach. Accident Analysis & Prevention, 176, 106795.

Howlader, M. M., Yasmin, S., Bhaskar, A., & Haque, M. M. (2023). A before-after evaluation of protected right-turn signal phasings by applying Empirical Bayes and Full Bayes approaches with heterogenous count data models. Accident Analysis & Prevention, 179, 106882.

Tahir, H. B., Yasmin, S., Lord, D., & Haque, M. M. (2023). Examining the performance of engineering treatment evaluation methodologies using the hypothetical treatment and actual treatment settings. Accident Analysis & Prevention, 188, 107108.

Khan, S. A., Tahir, H. B., Yasmin, S., & Haque, S. M. M. (2025). Clear zone or W-beam guardrail for rural highways? A Full Bayes before-after evaluation by employing the Poisson Gamma-Lindley model. Accident Analysis & Prevention, 222, 108218.

Elvik, R. (2002). The importance of confounding in observational before-and-after studies of road safety measures. Accident Analysis & Prevention, 34(5), 631-635.

Shen, Q. (2007). Development of safety performance functions for empirical Bayes estimation of crash reduction factors (Doctoral dissertation, Florida International University).

Mountain, L., Fawaz, B., & Sineng, L. (1992). The assessment of changes in accident frequencies on link segments: a comparison of four methods. Traffic Engineering & Control, 33(8).

Gross, F., Persaud, B. N., & Lyon, C. (2010). A guide to developing quality crash modification factors (No. FHWA-SA-10-032). United States. Department of Transportation. Federal Highway Administration. Office of Safety.

Abdel-Aty, M. A., Lee, C., Park, J., Wang, J. H., Abuzwidah, M., & Al-Arifi, S. (2014). Validation and application of highway safety manual (part D) in Florida (No. BDK78-977-14). Florida. Dept. of Transportation.

Lan, B., Persaud, B., Lyon, C., & Bhim, R. (2009). Validation of a full Bayes methodology for observational before–after road safety studies and application to evaluation of rural signal conversions. Accident Analysis & Prevention, 41(3), 574-580.

Hauer, E., Harwood, D. W., Council, F. M., & Griffith, M. S. (2002). Estimating safety by the empirical Bayes method: a tutorial. Transportation research record, 1784(1), 126-131.

Ko, M., Geedipally, S. R., & Walden, T. D. (2013). Effectiveness and site selection criteria for red light camera systems. Transportation research record, 2327(1), 53-60.

Persaud, B., & Lyon, C. (2007). Empirical Bayes before–after safety studies: Lessons learned from two decades of experience and future directions. Accident Analysis & Prevention, 39(3), 546-555.

Sacchi, E., & Sayed, T. (2015). Investigating the accuracy of Bayesian techniques for before–after safety studies: The case of a “no treatment” evaluation. Accident Analysis & Prevention, 78, 138-145.

Gooch, J. P., Gayah, V. V., & Donnell, E. T. (2016). Quantifying the safety effects of horizontal curves on two-way, two-lane rural roads. Accident Analysis & Prevention, 92, 71-81.

Ogundimu, E. O., Altman, D. G., & Collins, G. S. (2016). Adequate sample size for developing prediction models is not simply related to events per variable. Journal of clinical epidemiology, 76, 175-182.

Balusu, S. P. K., Iraganaboina, N. C., Shallam, R. D. K., & Chunchu, M. (2017). Speed-profile-based road segmentation for accident occurrence modelling for hilly terrains. International journal of injury control and safety promotion, 24(4), 444-451.

Alhadidi, T. I., Al-Marafi, M. N., & Alazimi, A. (2025). Development of safety performance measures for different crashes severity at urban roundabouts. Results in Engineering, 25, 103680.

Al-Ahmadi, H. M., Jamal, A., Ahmed, T., Rahman, M. T., Reza, I., & Farooq, D. (2021). Calibrating the highway safety manual predictive models for multilane rural highway segments in Saudi Arabia. Arabian Journal for Science and Engineering, 46(11), 11471-11485.

Zhang, P., Wang, C., Chen, F., Cui, S., Cheng, J., & Bo, W. (2022). A random-parameter negative binomial model for assessing freeway crash frequency by injury severity: Daytime versus nighttime. Sustainability, 14(15), 9061.

Wang, C., Chen, F., Cheng, J., Bo, W., Zhang, P., Hou, M., & Xiao, F. (2020). Random‐Parameter Multivariate Negative Binomial Regression for Modeling Impacts of Contributing Factors on the Crash Frequency by Crash Types. Discrete Dynamics in Nature and Society, 2020(1), 6621752.

Cafiso, S., Montella, A., D’Agostino, C., Mauriello, F., & Galante, F. (2021). Crash modification functions for pavement surface condition and geometric design indicators. Accident Analysis & Prevention, 149, 105887.

Subedi, A., Acharya, S., Singleton, P., & Mekker, M. (2025). Safety Performance Functions and Crash Modification Factors for Skid Resistance on Interstate and Non-Interstate Highways. Transportation Research Record, 03611981251337473.

Kumfer, W., Harkey, D., Lan, B., Srinivasan, R., Carter, D., Patel Nujjetty, A., ... & Tan, C. (2019). Identification of critical intersection angle through crash modification functions. Transportation research record, 2673(2), 531-543.

Al-Marafi, M. N., Somasundaraswaran, K., & Bullen, F. (2021). Development of crash modification factors for intersections in Toowoomba city. International Journal of Urban Sciences, 25(1), 104-123.

Khavarian, K., & Sahebi, S. (2025). Robust crash modification factor estimation with case-control method. Traffic Injury Prevention, 1-12.

Kay, J., Gates, T. J., Savolainen, P. T., & Shakir Mahmud, M. (2022). Safety performance of unsignalized median U-turn intersections. Transportation research record, 2676(9), 451-466.

Alhadidi, T. I., Al-Marafi, M. N., & Alazimi, A. (2025). Development of safety performance measures for different crashes severity at urban roundabouts. Results in Engineering, 25, 103680.

Vinayaraj, V. S., & Perumal, V. (2023). Developing safety performance functions and crash modification factors for urban roundabouts in heterogeneous non-lane-based traffic conditions. Transportation research record, 2677(8), 644-661.

Anjana, S., & LR Anjaneyulu, M. V. (2015). Development of safety performance measures for urban roundabouts in India. Journal of transportation engineering, 141(1), 04014066.

Saljoqi, M., Behnood, H. R., & Mirbaha, B. (2021). Developing the crash modification model for urban street lighting. Innovative infrastructure solutions, 6(2), 59.

Getnet, M., Bisht, L. S., & Tiwari, G. (2024). The safety impacts of paved shoulder width in Indian four-lane rural highways. International Journal of Injury Control and Safety Promotion, 31(4), 660-671.

Almasi, S. A., & Yang, J. (2026). A spatially adaptive empirical Bayes framework with dynamic dispersion parameters for enhanced crash frequency prediction across rural highway networks. Accident Analysis & Prevention, 226, 108361.

Noland, R. B., & Adediji, Y. (2018). Are estimates of crash modification factors mis-specified?. Accident Analysis & Prevention, 118, 29-37.

Galgamuwa, U., Du, J., & Dissanayake, S. (2021). Bayesian spatial modeling to incorporate unmeasured information at road segment levels with the INLA approach: A methodological advancement of estimating crash modification factors. Journal of traffic and transportation engineering (English edition), 8(1), 95-106.

Champahom, T., Se, C., Jomnonkwao, S., Kasemsri, R., & Ratanavaraha, V. (2023). Analysis of the effects of highway geometric design features on the frequency of truck-involved rear-end crashes using the random effect zero-inflated negative binomial regression model. Safety, 9(4), 76.

Al-Marafi, M. N., Alhadidi, T. I., Alhawamdeh, M., & Jaber, A. (2024). Enhancing Road Safety Strategies through Applying Combined Treatments for Different Crash Severity. Urban Science, 8(3), 109.

Ahern, Z., Corry, P., Shirazi, M., & Paz, A. (2025). A comprehensive multi-objective framework for the estimation of crash frequency models. Accident Analysis & Prevention, 210, 107844.

Fitzpatrick, K., Park, E. S., Cynecki, M. J., Pratt, M. P., & Beckley, M. (2021). Investigation of crashes at pedestrian hybrid beacons: Results of a large-scale study in Arizona. Journal of safety research, 78, 59-68.

Austroads. (2016). Guide to Traffic Management Part 4:  Network Management. https://austroads.gov.au/publications/traffic-management/agtm04

Austorads. (2021). Guide to Road Design Part 1: Objectives of Road Design. https://austroads.gov.au/publications/road-design/agrd01

Washington, S., Karlaftis, M. G., Mannering, F., & Anastasopoulos, P. (2020). Statistical and econometric methods for transportation data analysis. Chapman and Hall/CRC.

Lord, D. &. (2010). The statistical analysis of crash-frequency data: A review and assessment of methodological alternatives. Transportation research part A: policy and practice, 44(5), 291-305.